Operation Manual

# HFT11

# Hydraulic Crawler Excavator

Original Instruction

Shandong HFT Heavy Industry Machinery Co., Ltd.

Please read this manual and fully

understand its content before proceeding with the operation, inspection and

maintenance of this machine

Safety Warning Signs

includes

This sign denotes a safety warning. The information behind the sign

important safety information.

Please read and understand this information to prevent personal injuries or deaths.

The machine  owner  or  employer  is  responsible for   instructing   every  operator   to    operate   all equipment    correctly    and    safely    in     detailed manner.  All  personnel  operating  this  machine shall sufficiently understand the contents of this manual.

Before   operating    the   machine,    all   operation personnel  must  receive  the  guidance  on  related functions of excavator.

Before operating this machine in a working site, learn   and   practice   how  to  use  the  machine controls correctly in a safe and spacious place.

The     improper     methods     during     operations, checking,   and    maintenance   of   machine   will result in injuries or deaths.

Before any operation, checking, or maintenance of machine, please read and understand  this

manual.

Please  always  carry  this  manual with you.  It’s preferable    to    preserve    this    manual    on    the machine.  If  this   manual   is   lost   or   damaged, please immediately order a new manual  from a dealer.  At  the  transfer  of  this  machine,  please

ensure  to  hand  this  manual   over  to  the  new owner.

The   machine   supplied   by  Ken    stone   Heavy Machinery  conforms  to  the local  standards and  specifications  of the  country  exported.  If your  machine   is   purchased    from   other   country    or from an individual or company of other country,  this  machine  is  probably  not installed  with  the  safety  devices  or   safety  standards  required   for the operations of this machine in your country.  If you have any doubt on the compliance of  your machine     with     the     relevant     standards     and regulations in your country, please contact a  Ken  stone dealer.

Signal Words

The safety information on this manual and

machine identifications is indicated by words “Danger”, “Warning”, and “Notice” . The

meanings of these signal words are as below:

# ΔDanger

“Danger” denotes high dangerous level, for which the negligence will result in death or serious injury.

It’s impossible for us to predict all possibly

existed dangers. Therefore, the warnings in this manual or on this machine can’t cover all

possible accidents. Therefore, during the

operations of machine, ensure to take cautions and abide by all conventional safety measures to prevent harms to machine, operation

personnel, or other personnel.

ΔWarning

“Warning” denotes medium danger level, for which the negligence will probably result in   death or serious injury.

Notice

“Notice” denotes low danger level, for which  the negligence will result in minor or medium injuries.

Important: The word “Importance” is  intended

to alert the operation and  maintenance

personnel of the possible  damages of machine and its components.

0-1

Introduction

## Foreword

This manual describes the operations, checking, and maintenance of this machine and the  safety descriptions to be observed during  operations.   Should you have any doubt, please contact a

dealer or service provider of Ken stone Heavy

## Machinery.

●      In some details, this manual probably    differs from the manual attached on the machine in use.

●      Please be noted that the information contained in this manual and the

parameters of machine are subject to changes without further notice.

The manual is in the circular manual box behind the seat

Serial number

Important:  Do  not  disassemble  the  machine nameplate  with   serial  number.  Observe  the serial numbers of machine and engine and

record them in the blank area below.

Machine No.:

Engine No.:

0-2

Introduction

Front, Rear, Left, and Right

Right

Front

Rear

Left

The front, rear, left, and right of the machine

indicated in this manual are based on the status sitting in the driver seat, with the bulldozing

blade visible in the front.

## Designated Operations

This machine is mainly applied for following operations:

●     Digging

●      Ditching

●      Side ditching

●      Leveling

●      Loading

## Product Features

●      Small size, flexible operation

●      Can be equipped with a variety of attachments

●      Track width "flexible machine width structure"

Run-in period

For the first 100 hours of the new machine (as indicated by the timer), please follow the

instructions below:

Using the new machine without run-in will deteriorate acceleration performance and     probably shorten the life of machine.

●      Sufficiently warm up the engine and hydraulic oil.

●      Avoid heavy-load and rapid operations.

Maintain the load at approximately 80% of maximum load capacity during operations.

●      Do not start, accelerate, steer, or stop suddenly, unless it’s really necessary.

0-3

Introduction

## Notes about Reading of This Manual

Please be noted that the descriptions and

graphics contained in this manual probably are not  applicable  for  your  machine  in  use.  The numbers in graphics are marked by circles. The same numbers in the descriptions are marked in parentheses. (For instance: ①→(1))

Signs used in this manual

The meanings of the signs used in this manual are as below.

,

… . Prohibition

… … . Lock

……Unlock

## Noise and vibration

A-Weighted emission sound pressure level at the operator's position measured according to work cycle procedures specified in “ISO

6396 “ is 78dB(A), The hands and arms are exposed to a weighted root mean square

acceleration that is less than 2.5 m/s2.

The body is exposed to a weighted root mean square acceleration that is less than 0.5 m/s2.

Measurements are obtained on a

representative machine using the procedures in the following

standards: ''ISO 2631-1" ''ISO 5349-1" ''ISO 5349-2"

0-4

Contents

Signal words	0-1

Introduction	0-2

Safety	1-1

General Precautions	1-2

Precautions during Preparations	1-8

Safety Measures at Start	1- 10

Operation Precautions	1- 12

Precautions for Parking	1-22

Precautions during Transport	1-23

Precautions for Maintenance 	1-25

Safety Signs 	1-35

Controls	2-1

Component Name	2-2

Cover(Start Key / engine cover / fuse case / fuel

filler port)	2-4

Seat	2-6

Electrical control panel	2-8

Switches (start switch / horn button)	2- 10

Joysticks	2- 11

Accessory	2- 14

Operations	3-1

Before Operations	3-2

Start and Stop of Engine	3-3

Operate Machine	3-7

Operations Procedures	3- 17

Parking of This Machine	3-26

Treatment in Cold Weather	3-28

Handling of Rubber Tracks	3-29

Transportation	4-1

Loading and Unloading	4-2

Lifting of Machine	4-4

Fixing of Machine	4-6

Maintenance	5-1

Overview	5-2

Service Data	5-4

Critical Safety Parts	5- 11

Maintenance List	5- 13

Walk-Around Checking	5- 15

Daily routine checking (Every 10h)	5- 17

After first 50h (For new machine only)	5-23

Every 50h	5-

25

Every 250h	5-30

Every 1000h	5-35

Every 1500h	5-39

Every 2000h	5-40

As necessary	5-42

Maintenance during Long-Time Parking	5-49

Troubleshooting	6-1

Symptoms that are not malfunctions	6-2

Engine Overheating	6-3

No Battery Power	6-4

Burnout of Fuse	6-6

Restart after Refueling	6-8

Flashing of Warning Lamps	6-9

Other Symptoms	6- 10

Towing	6- 12

0-5

Parameters	7-1

Basic Parameters	7-2

Overall Dimensions of Machine	7-

3

Operating Range	7-4

Machine lifting power	7-5

Accessories	8-1

General Safety Measures  	8-2

Precautions	8-5

Hydraulic Hammer 	8-6

Optional accessories	8.11

Parts number position	8- 13

0-6

# Safety

1-1

# Safety

General Precautions

General Precautions

You are liable to abiding by the safety  laws   and regulations of relevant departments and fulfilling the operations, checking, and

maintenance of machine.

As a matter of fact, all accidents are caused by the non-compliance with basic safety rules and precautions.

Most accidents can be avoided by  authenticating the potential dangers in  advance.

Please read and understand all safety

information related to the prevention of

accidents. Please ensure to operate the  machine only after you have understood how  to operate, check, and maintain the machine  correctly.

Abiding by All Safety Regulations

●      The machine must be operated, checked, and maintained by trained and qualified   personnel.

●      During the operations, checking, and maintenance of machine, ensure  to    understand and abide by all rules,

regulations, precautions, and safety

measures.

●      Do not operate, check, or maintain the   machine under the adverse influence of alcohol, drug, medicine, or fatigue  or

under sleepy status.

Upon detection of machine abnormality During the operations, checking, or

maintenance of machine, upon detection of any machine abnormality (Such as noise,  vibration, smell, abnormal instrument, smoking  or oil

leakage, error warning indication, and  abnormal display of electric control panel),  immediately

notify a sales or service dealer and  take    appropriate measure. Do not operate the    machine before the abnormality is solved.

Operating Temperature Range

To maintain the performances and prevent the earlier wear of machine, please abide by the    following operating conditions.

●      Do not operate the machine when the

outside temperature is more than+45℃ or less than-15℃

.      The operations under+45℃ outside

temperature will probably result in engine overheating and deteriorated engine

performance. In addition, the hydraulic oil will probably become really hot to harm    the hydraulic devices.

.      The operations under 15℃ outside

temperature will probably harden the

rubber parts (Such as gaskets) and cause earlier wear or damage of machine.

.      To operate the machine under the

1-2

Safety

General Precautions

condition beyond above-mentioned  outside temperature range, please consult  with

your sales or service dealer.

Wearing Appropriate Clothing and Protective Appliances

●      Do not wear loose clothing or wear

decorative articles that will probably hitch to any joystick or motion part.

●      Do not wear oil or fuel contaminated

clothing that is easily vulnerable to fire.

●      As per the requirements of working

environment, wear safety shoes, safety

helmet, safety goggles, filtration mask,

thick gloves, ear flaps, and other protective appliances. During the use of grinding

miller, breaking hammer, or compressed  air, please wear appropriate protective     appliances, such as safety goggles and     filtration mask, as the splashing of metal chippings or other objects will probably

cause serious harms.

●      Please use the hearing protection devices

# during the operations of machine.

Exposure to a high noise environment for a long time will result in harm or even

complete loss of hearing.

Installation of Fire Extinguisher and First-Aid Kit

Fulfill the preparations for fire and accident.

●      Install the fire extinguisher and first-aid kit and learn how to use them.

●      Learn how to extinguish fire and handle accident.

●      Know how to contact emergency aid and fabricate the emergency contact list.

1-3

Safety

General Precautions

No Disassembling of Safety Devices

●      Please ensure that all guardrails, shelters, and doors are installed properly and

securely. Before operating the machine, please repair or replace damaged parts.

●      Know how to use safety lock handle, seat belt, and other safety devices and

understand the correct operation methods.

●      Do not disassemble any safety device,  unless for the purpose of checking and repair. Always keep all safety devices   under good working condition.

Setup of Signaler and Flagman

responsible for making gestures.

●      All personnel must completely understand all gestures.

●      The  operator must respond to the gestures of designated  person  only.  However,  the operator  must  abide  by  the  stop  gesture made by anyone in all cases.

●      The signaler must stand in a clearly visible place while making the gestures.

Precautions for Standing up from or Leaving Driver Seat

●      Before standing up from the driver seat to

Learn how to use the gestures of specific operation needs and designate the person

open/close windows or disassemble/install the lower window, lower the working

device onto the ground, lift up and lock the safety lock handle, and stop the engine.

When the safety lock handle is lowered     down (Unlocked), the accidental touch of any joystick will result in sudden

movement of machine and cause serious injuries or deaths.

1-4

# Safety

General Precautions

●      Please be noted that, even if the safety

lock handle is placed at locking position,  the bulldozing blade, boom, and auxiliary hydraulic controls can’t be locked.

Do not touch such controls accidentally.

●      While lifting up or lowering down the   safety lock handle, take cautions not to touch any joystick.

Before leaving the driver seat, lower the

working device onto the ground, lift up the

safety lock handle to locking position, and stop

the engine. Meanwhile, ensure to withdraw the

key, close the doors and hoods, and carry the key with you and then preserve it in a

designated place.

Guard against Fire and Explosion Dangers

Keep fuel, lubricating oil, grease, and antifreeze away from flame. The fuel is especially

inflammable and really dangerous.

●      Handle these inflammable away from

ignited cigarette, match, lighter, and  other

flame or fire source.

●      Do not smoke or use open fire while

handling fuel or performing the operations of fuel system.

●      Do not leave the operation site while filling fuel or lubricating oil.

●      Do not disassemble the fuel tank cap or

refuel while the engine is running or is not cooled down. In addition, do not splash

fuel to any machine hot surface or electronic system part.

●      Immediately clean the overflown fuel or lubricating oil thoroughly.

●      Check for leakage of fuel and lubricating oil. Please eliminate the leaks and clean   the machine before operations.

●      Please move the inflammable to a safe place before polishing or welding

operations.

●      Do not cut or weld any pipeline or pipe

that probably contains inflammable liquid. Please clean thoroughly by  non-

inflammable solvent before cutting or welding.

●      Remove all wastes and impurities from

machine. Ensure that there is no oil

contaminated rag or other inflammable on the machine.

●      Handle all solvents and dry chemicals

1-5

Safety

General Precautions

(Foam fire extinguisher) as per the

manufacturer’s procedures indicated on

the containers. Operate in a well-ventilated place.

●     Never use fuel for cleaning purpose.    Always use non-inflammable solvent.

●      Please open doors and windows for

thorough ventilation during the handling of fuel and the cleaning of oil stain or

paint.

●      Preserve all inflammable liquids and   materials in a safe and well-ventilated place.

●      The short-circuit of electric system will probably result in fire. Daily check the  wire connections for looseness and

damage. Re-tighten loose connectors and cable clamps. Repair or replace damaged wires.

●      Fire accident caused by pipelines: Ensure that the clamps, protection devices, and

cushion pads of hoses and pipes are

securely fixed. In event of looseness, the

hoses and pipes will be damaged due to

vibration or contact with other parts during operations. This will probably result in

spray of high pressure oil to cause fire accident or harms.

· Toxic Exhaust Gas from Engine            ·

●      Do not  operate the  engine in  an  enclosed place with poor ventilation.

●      If the  natural  ventilation  is  not  possible, install    ventilation     fan,    fan,    extended

exhaust pipe, or other ventilation device. Handling of Asbestos Powders

The inhalation of asbestos powders will

probably cause lung cancer. While handling  the materials probably containing asbestos, take the following safety measures:

●      Do not sweep by compressed air.

●      Avoid polishing or grinding asbestos-contained parts.

●      During cleaning, use a vacuum device     installed with high-efficiency particulate air filter (HEPA).

●      If there is no other method for control of powders, please wear the specified

respirator. During indoor operations,    please install a ventilation system with polymer filter.

1-6

Safety

# General Precautions

| ● ● | During operations, the non-authorized personnel are prohibited to access the working zone. Please strictly abide by the rules and |  |
| --- | --- | --- |

environmental standards applicable for this working zone.

Be careful to avoid crushing or cuts

Never place your hand, foot, or other body parts  between the superstructure and chassis or tracks, between machine body and working device, or    between cylinder and motion parts. During

movement of machine, these clearances will change to probably cause serious injuries or deaths.

Use of Accessories

●      Please consult with Ken stone company

before installing accessories. Based on the attachment types  and their combinations,   the attachments  will probably come into

contact with the  cab or other machine

parts. Before use,  ensure that the installed   accessories  will not come into contact with other parts.

●      Do not use any attachment not approved    by Ken stone company. Otherwise,  it will probably endanger the safety or  cause

adverse influence on operations or  life of machine.

●      Ken stone company will not be  liable for any injury, accident, or product  damage    arising from the use of  non-approved

attachments.

No Modification to Machine

The non-authorized modifications to the machine will cause injuries or deaths.

All non-authorized modifications are prohibited for any part of machine.

1-7

# Safety

# Precautions during Preparations

Precautions during Preparations Understanding of working zone

Before starting operations, understand the  situation of working zone to ensure safety.

●      Check the terrain and ground situation of working zone. For indoor operations,

check the building structure and when necessary take safety measures.

●      Ensure to keep away from dangers and

obstacles such as water ditch, underground pipelines, trees, cliffs, overhead cables,

and roads with falling stone or landslide danger.

●      Check the positions of underground gas pipelines, water pipelines, and electric

cables together with administrator. If

necessary, discuss with administrator to    determine the detailed safety measures to be taken to ensure safety.

●      During operations on road, ensure to take    the safety of pedestrians and vehicles into

consideration.

.      Use a signaler and/or signals.

.      Isolate the working zone against the access of non-authorized personnel.

●      Before   operations   in   water   or   driving through  shallow  rivers,  check  the  water depth,  ground  firmness,  and  flow  rate  in advance.

Understand more operation description

information with reference to the “Precautions during Operations”

Checking of Bridge Strength

Please check the permissible load before

driving on bridges or structures. If the  strength

is insufficient, reinforce the bridges or structures.

1-8

Safety

Precautions during Preparations

Always Keeping Clean Machine

●      Eliminate all loose  objects and

unnecessary devices in the machine.

●      Wipe away lubricating oil, grease, mud,

snow,  and ice, in order to prevent accident due to slip.

●      Remove dusts, lubricating oil, and grease from engine portion to prevent fire

accident.

●      Clean the area around operator's seat and remove all unnecessary objects from the  machine.

Daily Checking and  Maintenance

The failure to find out or repair the

abnormalities or damages of machine will cause accidents.

●      Before operations, fulfill the designated checking and when necessary repair

immediately.

●      In event of operation failure due to

malfunction or engine malfunction,

immediately stop the engine as per stop    procedure and park the machine securely, till the malfunction is repaired.

Precautions in the driver's cabin   If the machine is equipped with a driver's cab:

●   Before entering the driver's cab, please

remove the dirt and grease from the soles of your shoes. Operating the machine

pedal with dirt and grease adhering to the sole can cause an accident due to slipping under your feet.

●   Do not place parts or tools around the driver's seat.

●   Do not place any plastic bottles in the cab or install any suction cups on the window glass. The plastic bottle or suction cup

acts as a lens and may cause a fire.

●   Do not use your mobile phone while walking or working.

●   Do not bring combustibles or explosives into the operation area.

●   Do not leave the lighter in the driver's

cabin. When the room temperature rises, the lighter may explode.

1-9

Safety

Safety Measures at Start

Safety measures when starting

Support your body weight by three-point  safety

posture while getting on/off the machine.

●      Do not jump on or off the machine. Do not attempt to get on or off a moving machine.

●      Before getting on or off the cab, firstly

open the door completely to  locking

position and check and ensure that the door can’t move further(For machine with cab).

●      Face towards the machine, get on/off the     footplates, and grab the handrail to support your weight by three-point safety posture    (Hands and feet).

●      Do not use safety lock handle or any joystick as handrail.

Before starting machine, keep all

non-authorized personnel away from this zone

Start the engine only after the safe start is    confirmed by checking the following items.

●      Walk around the machine and alert the

repair personnel and the personnel walking

around the machine. Start the engine only after it’s confirmed that there is no person around the machine.

●      Check for warning sign “No Operation” or similar sign affixed on cab door, control,    or starter switch. If present, do not start

engine or touch any joystick.

●      Sound the horn to alert the personnel around machine.

Sit in the driver seat and start the engine.

●      Adjust the seat till it’s securely locked.

●      Fasten the seat belt.

●      Check whether the parking device is

1-10

Safety

Safety Measures at Start

actuated and all joysticks and pedals are in neutral.

●      Check whether the safety lock handle is at locking position.

●      Ensure there is no person near the machine.

●      Start and operate the machine only in driver seat.

●      Do not attempt to start the engine by short circuiting the terminals of starter device.

Start by Jumper Cable

Start by jumper cable only as per the

recommended method. The improper use of

jumper cable will result in battery explosion or unexpected machine motions.

Please refer to “In event of no battery power” to

understand more operation description information.

After Start of Engine

After the start of engine, fulfill following

operations and checking in a place without    personnel or obstacle. Upon detection of any malfunction, stop the engine as per the

procedure and report malfunction.

Warm up the engine and hydraulic oil.

Check all instruments and warning devices for normal functioning.

Check for presence of noise.   Test the engine speed control.

Operate all control devices to ensure normal functioning.

For Cold Weathers

Take cautions that the frozen ground, footplates, and handrails are slip. Under cold weathers, do   not touch any metal part of machine by bare

hands. Your skin will be frozen on metal part to cause serious injuries.

Do not use ethyl ether or starting fluid on the   engine. The starting fluid can cause explosion and serious injuries or deaths.

Warm up the engine and hydraulic oil.

Operating joysticks without warm-up will result in slow or inappropriate reaction or movement    of machine to cause accident.

1-11

Safety

Operation Precautions

Operation Precautions

Operating Machine with Maximum Track Width

Always operate the machine with extended

track width of 950mm, in order to improve  the machine stability to the maximum extent.

Operating the machine with narrow track width (800mm) will probably result in rollover of

machine due to poor stability.

If it’s necessary to operate the machine with

narrow track width (800mm), retract the bucket working device and lower the boom to reduce    the gravity center and face the machine towards front before traveling.

Guarantee of Good Visibility

Checking of Visibility before Operations of Machine

During operations in a dark place, turn on the working lamp and headlamp of machine and

when necessary install additional  lighting

devices. In event of poor visibility due to severe

weathers (Such as fog, snow, rain, and haze),     stop the operations of machine till the visibility turns good.

Clean the windows, mirrors, lamps, and camera to keep good visibility. Adjust the mirrors and   camera to optimal positions to ensure that the    driver sitting in the driver seat can see the rear   view (Blind spot).

The non-authorized modification or the

installation of non-approved attachments will   probably impair the visibility. The operator’s   visibility must conform to the requirements of

ISO 5006.

Do not carry people on the machine

All personnel are prohibited to sit on any part of machine at any time during traveling or

operations of machine.

1-12

Safety

Operation Precautions

Checking for Safe and Reliable

Working Zone before Operations

Verify the performance limits of machine.

Set up a signaler at the road shoulders, narrow places, or vision-obstructed places.

Do not allow any person to access the turning radius or path of machine.

Sound the horn to indicate your movement intention.

There are blind zones behind the machine.

Before driving backward, check the safety in     the area behind machine and ensure there is no

person behind the machine.

Checking of Chassis (Tracks) before  Traveling

Before operating the traveling joystick/pedal, ensure that the bulldozing blade is in front of the driver seat. Please bear in mind that the     operation direction of the traveling

joystick/pedal with bulldozing blade behind the driver seat is reverse to that with bulldozing

blade in front of driver seat.

Safe Traveling

The bulldozing blade shall be lifted during

traveling. Retract the bucket working device as shown in above diagram and lift the bucket by  30~40cm off the ground.

Do not slew during the traveling. If it’s

necessary to operate the bucket working device during traveling, operate at sufficiently low

speed to ensure the complete control at all times.

1-13

Safety

Operation Precautions

●   Avoid driving over obstacles whenever

possible.  If it’s necessary to drive over

obstacles, keep  the bucket operating device  near the ground and  travel at low speed. Do not drive over any  obstacle that will incline the machine by 10° or  larger.

●   On rugged roads, drive the machine at low     speed and avoid the sudden startup, stop, and direction change. Otherwise, the working

device will probably come into contact with the  ground to cause unbalance and damage  of  machine or destruct the structure in

surrounding  area.

Precautions for Traveling on Slopes

While traveling on  slopes,  take  cautions to prevent the rollover and gliding of machine.

●   Do not drive the machine on a slope

steep  enough to cause instability of

machine. Please  be noted that, in the

actual applications, the  performances of machine on a slope will be   deteriorated  due to severe working conditions.

Bulldozing blade

●   Keep the driver seat facing towards the up slope  direction while climbing up a slope. Keep the  driver seat facing

towards the down slope  direction during down slope traveling. In both  cases, pay  special attention to the ground ahead  of

the machine during traveling.

●   While  traveling on a slope, lower the bucket to  20~30cm off the ground. While climbing  up a steep slope, extend the bucket working  device to the front position. In event of an

emergency,  lower the bucket onto the     ground and stop the   engine of machine.

●   When walking on slopes or ramps, drive   slowly. Reduce engine speed when going downhill.

●   Do not reverse the  machine down a slope.

1-14

Safety

Operation Precautions

●   Do not change direction on slopes or

cross slopes. First return to the flat surface, and then take another path.

●   When walking on a gentle slope covered with grass or dead leaves, or when

walking on wet metal plates or frozen

ground, the machine may slip. Make sure   that the machine does not stop laterally on

·       a slope.                                                              ·

●   If the machine stalls on a slope, return the operating levers to the neutral position,

and then restart the engine.

Special Cautions for Operations of Machine  on Ice/Snow

●   While traveling on snowy or icy roads, drive at  low speed and avoid the sudden startup,    stop,  and direction change.

●   In snowy regions, the road curbs and the

roadside objects are buried in the snow to become invisible. In addition, there is a

danger of machine rollover or collision with covered  objects. Therefore, always operate  with  cautions.

●   There is a danger of rollover or snow

entrapment while driving the machine into    thick  snow. Drive with cautions and do not  drive over  road curbs or entrap in the snow.

●   For frozen soil surface, the ground becomes soft when the temperature rises, which will   probably result in rollover of machine and    entrapment of operator in the machine.

●   While parking the machine on a unstable ground, lower the bulldozing blade.

1-15

Safety

Operation Precautions

No Movement of Bucket over Personnel

Moving the bucket over the personnel has a   danger of splashing of loaded material or the sudden falling of bucket.

Guarantee of Driver’s Safety during Loading

Do not load before the driver reaches a safe place.

●   Do not swing or position bucket above personnel or cab.

●   Load from the backdoor of  trucks.

Keeping a Safe Distance from Overhead High-Voltage Cables

Never close any part or loaded material of

machine to high-voltage cables, unless all

safety measures specified by local and national authorities are already taken. There is a danger of electric shock and death when any person

approaches to the machine that is discharging electric spark or is near or in contact with

power supply.

●   Always keep a safe spacing between machine  and high-voltage cables.

●   Before starting operations, please verify the relevant operation safety procedures with     local power grid.

●   Consider all electric cables as high-voltage  cables. Even if it’s known or believed that    the  power supply is cut off or the cable is    clearly  grounded, consider such cables also as live  cables.

●   If the machine is too close to  high-voltage

1-16

Safety

Operation Precautions

cables, set up a signaler to sound warnings.

●   Keep all personnel within operation zone  away  from machine and loaded material.

●   Pay special attention to the underground high-voltage cables.

Close Observation on Dangerous Working Conditions

●   Never dig on the bottom of any high

embankment. It’s really dangerous because such operation will probably result in

surface  collapse.

●   Do not operate in any place with falling stone  danger.

●   Keep a safe spacing between machine and    edge  of digging site. Do not dig the ground ahead of  machine.

●   During the operations near a cliff or road

curb,  to ease the escape in any event, form a right-angle between tracks and cliff or road    curb and place bulldozing blade in the front.

●   Do not access any soft ground area. This will  probably result in inclination of

machine due to  dead weight to cause rollover or ground  entrapment.

1-17

Safety

Operation Precautions

●   Keep away from unstable ground (Cliffs,

road  curbs, and deep ditches). If the ground collapses due to machine weight or

vibration,  there is a possible danger of machine falling or  rollover.

•     Please bear in mind that the soil turns infirm after heavy rain or explosion.

•     The ground on the top of embankment and the  ground around and on top of the dug    ditches are  also infirm.

●   Do    not     perform     demolition     operations beneath     the  machine.   There  is   a  possible falling danger   of machine when  the ground becomes unstable.

●   Before the operations on the top of any

building  or other structure, check the

strength and  structure. It will cause serious harms or  destruction if the building or

structure  collapses.

●   Do not perform demolition operations above your head. There is a falling damage of

damaged parts or collapse danger of building to  cause serious harms or    destruction.

●   Do not perform the demolition operations by the impact force of bucket working device.    The  splashed material fragments and the

damaged  bucket working device have a danger of serious harms.

1-18

Safety

Operation Precautions

Danger during Operations on Slopes

During operations on slopes, slewing or

operating the working device can probably

result in instability or rollover of machine.

Avoid operations on slopes whenever possible.

Landfill

●   Leveling operation zone

●   If the bucket is fully loaded with material, avoid slewing towards the down slope

direction.  Otherwise, it will deteriorate the machine  stability and probably result in

rollover.

No Lateral Slewing (Swing) with Bucket Heavily Loaded

The machine is more vulnerable to rollover in lateral direction than longitudinal direction.

●   Do not laterally slew (swing) when the

bucket  working device is heavily loaded.     Especially,  do not laterally slew (swing) on a slope.

●   When the machine is equipped with

breaking  hammer, grinder, or extended

bucket arm, the  attachment end is heavier     than that with  standard bucket. For machine with such heavy  end, do not face the bucket arm (boom) towards  down slope direction    for digging or towards  lateral direction for    operations.

1-19

Safety

Operation Precautions

Please pay attention to the objects above your head

During operations under bridges, in tunnels,   near cables, or indoors, guard the boom and    bucket arm against impact with any overhead object.

Not Designed for Lifting Application

This machine is specially designed for digging operations. Therefore, it’s not installed with

safety devices for lifting operations. Take

special cautions while using this excavator for lifting operations.

●   Do not overload during lifting. The overload will result in rollover of machine to  cause

serious injuries or deaths.

●   All rated lifting capacities are based on the

condition of using this machine on a firm and level ground. The user’s is recommended to

reserve an appropriate allowance for specific

working conditions, in order to ensure the safe lifting operations. The soft or uneven ground,   non-level status, lateral load, dynamic or

sudden load, dangerous environment, and

personnel experience are included. Before

operating the machine, the operation personnel  and other personnel shall be really familiar with the Operation Manual. The safe operation

regulations of the machine shall be strictly followed in all cases.

●   If the chain or lifting device is incorrectly

connected, the bucket connecting rod or the

lifting device will probably fail to cause serious injuries or deaths.

●   While using this machine as a crane, do not    attempt to pull out a stump from the ground.   For this application, the load applied onto the machine is completely unknown.

●   Do not allow any person to stand on or beneath the lifted object or close to the operation zone.

Please pay attention to flying objects

This machine is not installed with any   protective  device to guard the operator against  the harms of flying objects. Do not operate this

1-20

Safety

Operation Precautions

machine in any dangerous place in which the operator is probably subject to the impact of  flying objects.

Precautions for Towing

Please refer to the section “Towing” to

understand more description information.

20°or less

During towing, the improper operations, the

incorrect use of rope, or the improper checking will cause serious injuries or deaths.

●   The breakage or cracking of rope can cause danger. Use wire ropes compatible with the towing force.

●   Do not use any kinked, twisted, or even damaged rope.

●   Do not suddenly apply heavy load onto the

rope.

●   Please wear safety gloves during the handling  of rope.

●   Ensure that an operator is assigned each on    the  towing machine and the towed machine.

●   Do not operate the towing operation on a slope.

●   Keep all personnel away from the rope

during  towing.

1-21

Safety

Precautions for Parking

Precautions for Parking Safe parking

●      Stop the machine on a level, solid, and  safe ground. Set the parking device.

●      Complete the following operations before leaving the machine:

Lower the bucket and bulldozing blade onto the ground.

Lift up the safety lock handle to the locking position.

Stop the engine and withdraw the starter key.

Lock the cab and hood and carry the key with you.

●      If it’s necessary to park the machine or tilt the machine on a slope, securely park the   machine and block the machine against

movement.

While parking the machine in a street, use gratings, warning signs, or lamps to keep   the visible even in the night, in order to

prevent the impact with other vehicles.

1-22

Safety

Precautions during Transport

Precautions during Transport Safe loading/unlocking of machine

Fix to suspension parts

Distance between

Stop block

15°or less

Ramp

The machine will probably roll over or fall off during loading and unloading. Ensure to take    the following safety measures:

Select a solid and level ground and keep a sufficient spacing from the road curb.

Fix the ramps of sufficient strength and

dimensions to the cargo body of truck. The

inclination of the ramps shall not exceed  15°. If  the ramps deflect downward excessively, please support the ramps by supports or cushion

blocks.

Do not load or unload the machine by working device. Otherwise, it will probably result in

rollover or falloff of machine.

Keep the truck cargo body and the ramps clean   without oil, sand, ice/snow, or other impurity, in order to prevent the side slip of machine. Clean  the tracks.

Block the wheels of transport truck  by wedges

to prevent movement. While loading or

unloading the machine, drive the machine

slowly in 1st gear (low speed) as per the signals of the signaler.

Do not change direction on ramps.

Do not slew/swing on ramps. Otherwise the machine will probably roll over.

Slewing (swinging) the machine on the cargo body of truck will probably result in unstable legs of machine. Therefore, operate slowly.     If possible, lock the cab doors after loading.   Otherwise the cab doors will probably open    during the transport.

Plug securely the tracks by wedges and then fix the machine to the truck cargo body securely by ropes or chains.

1-23

Safety

Precautions during Transport

Safe Lifting of Machine

Master and utilize the correct lifting gestures.    Daily check the lifting device for damaged and missing parts and when necessary replace.

Please use the ropes of sufficient capacity for the machine weight during lifting.

Lift the machine as per the procedure described below. Do not operate by any other method,

otherwise it will probably result in unbalance of machine.

Please refer to the section “Lifting of Machine” to understand more description information.

Do not operate the lifting if there is any operator on the machine.

Operate slowly during lifting to prevent the rollover of machine.

During lifting, keep all personnel away from

working zone. Do not move the machine over any person.

Safe Transport of Machine

During the transport of machine, understand

and abide by all applicable safety rules, vehicle codes, and traffic regulations.

Take the length, width, height, and weight of the transport truck with loaded machine into  consideration, in order to select  the best

transport route.

During the transport, do not start up or stop the  transport truck suddenly or drive at high speed. Otherwise, it will result in movement or

unbalance of loaded machine.

1-24

Safety

Precautions for Maintenance

Precautions for Maintenance Warning Information of “No

Operation”  Sign

During the checking or maintenance of

machine, the start of engine or the touch of any joystick by non-authorized personnel will

probably result in serious injury accidents.

Before maintenance, please stop the engine  and withdraw and carry the key.

Affix warning information “No Operation ” to an eye-catching place such as starter

switch and joystick.

Use of Correct Tools

Do not use any damaged or deteriorated tool or any  tool   designed   for   other   application.   Use tools suitable for related operations.

Periodically Replaced Critical Safety Parts

Replace the fuel hoses periodically. The fuel

hoses will wear gradually along with time, even if no wear symptom is visible.

Upon  detection  of  any  wear  symptom,  replace the   fuel   hose,   regardless   of   the   replacement schedule.

To understand  more  details,  please  refer  to  the section “List of Critical Safety Parts” .

Explosion-Proof Lamps

While checking the fuel, lubricating oil,

coolant, and battery electrolyte, please use

explosion- proof lamps to prevent fire and

explosion. Otherwise, it will probably result in explosion to cause serious injury accidents.

1-25

Safety

Precautions for Maintenance

No Access of Non-Authorized Personnel

During operations, the non-authorized

personnel are prohibited to access the working zone. Take cautions during grinding, welding,  and use of hammer. You  will probably be

injured by the flying fragments from the machine.

Preparations of Working Zone

Select a stable and level working zone. Ensure the appropriate illumination conditions. For

indoor operations, keep well ventilated.

Remove obstacles and dangerous goods. Clear slippery areas.

Always Keeping Clean Machine

Please clean the machine before maintenance.

Stop the engine before cleaning the machine.

Cover the electric parts against water ingress.

The water ingress into the electric parts will

probably result in short-circuit or malfunction.    Do not clean the battery, electronic control units, sensors, connector, or cabby water or steam.

Stop of Engine before Maintenance

While the machine is working or the machine is not working but the engine is running, avoid

lubricating or further adjusting the machine.

If the maintenance requires the running of

engine, assign two operators for teamwork and keep contact with each other.

One operator must sit in the driver seat to get ready to stop the engine immediately when     necessary. This operator must pay special

attention not to touch any joystick or pedal, unless it’s absolutely necessary.

The other operator for the maintenance must    keep the body and clothing away from motion parts of machine.

1-26

Safety

Precautions for Maintenance

Keeping Away from Motion Parts

Keep away from all rotating and motion parts. The entanglement of hands or tools into

rotating or moving parts will probably cause

accidents of serious injuries or even deaths.

The tools or other objects fallen or inserted into the fan or fan belt will be crushed or cut. Never throw or insert any object into fan or fan belt.

Secure Fixing of Machine and Possibly Falling Parts

Before the maintenance or repairs beneath the machine, lower all movable working devices   onto the ground or lowest position and fix the

tracks.

If it’s necessary to operate beneath the lifted machine or device, always fix by cushion

woods, jack, or other firm and stable supports. Do not access the area beneath the machine or  working device before it’s firmly supported.     This operation is especially important for the    hydraulic cylinder operations.

Fixing of Working Device

During the repairs and replacements of bucket teeth or side teeth, to prevent the accidental

movement of machine, securely fix the working device.

Steady Placement of Opened Engine Hood  and Cover

Before operations within the machine, please

securely fix the engine hood or machine hood. Keep the engine hood or cover closed under

windy weather or while parking the machine on a slope.

Placement of HeavyWeights at Steady

Place

If it’s necessary to place temporarily heavy

1-27

Safety

Precautions for Maintenance

weights or attachments onto the ground during disassembling or installation, please ensure to   place them in a steady place. Keep the  non-

authorized personnel away from the place   for storage of such objects.

Precautions for Refueling

The smoking and open fire are prohibited

during refueling and near the refueling point.  Do not disassemble the fuel tank cap or refuel while the engine is running or is not cooled     down. Do not splash fuel to any high

temperature surface of machine.

Refuel the fuel tank in a well-ventilated place.     Do not top up the fuel tank. Leave an  expansion

pressure in the fuel tank.

Do not use fuel for any cleaning purpose.    Use correct fuel grade based on the season.

Handling of Hoses

The leakage of lubricating oil or fuel can result in fire accident.

Do not distort, bend, or impact any hose.

Do not use any distorted, bent, or cracked

pipeline, metal pipe, or hose, otherwise it will probably result in burst.

Re-tighten any loose connector.

Absolution Caution during Handling of High  Temperature and High

Pressure Parts

space for the fuel.

Any overflown fuel shall be wiped away immediately.

Tighten the fuel tank cap securely. If the fuel   tank cap is missing, replace with original cap  only. The use of non-authorized fuel tank  cap with poor ventilation will cause  internal

Please stop the engine and wait for the machine to cool down before maintenance.

The engine, exhaust pipe, radiator, hydraulic

pipes, sliding parts, and many other machine parts are really hot when the engine is just

stopped. Touching such parts can cause

scalding. The engine coolant, hydraulic oil, and

other 1-28

Safety

Precautions for Maintenance

fluids are also under high temperature and high pressure status.

Take cautions not to touch the hydraulic oil   while loosening engine hood or connector.    Operating the machine under such condition will cause out-spray of hot oil to result in

scalding or injuries.

Absolution Caution during Handling

of High  Temperature and High Pressure Parts

Do not disassemble the radiator cap or drainage plug when the coolant is hot. Stop the engine

and wait for the engine and coolant to cool

down. Slowly loosen the radiator cap to relieve the internal pressure and then take it out.

Caution against Internal Oil Pressure

Take caution against internal oil pressure. After the stop of engine, the pressure in the hydraulic oil pipes can hold for a long time.

Before the maintenance, thoroughly relieve the internal pressure.

The high pressure of hydraulic oil can penetrate skin and eyes to cause serious injuries and

blindness or even death. Please bear in mind    that the hydraulic oil permeating from orifices is nearly invisible to naked eyes. While

checking for leakage, wear goggles and thick   gloves and protect the skin by paper boards or plywood to protect against the harms of

spraying hydraulic oil.

The hydraulic oil penetrating into your skin

must be cleared by a doctor familiar with such injuries with surgical method within several

hours.

Pressure Relief before Operations of Hydraulic System

Disassembling cap or filter or disconnecting     pipeline before the pressure relief of hydraulic system will probably result in out-spray of

hydraulic oil.

●      Slowly loosen the bleeding plug to relieve the pressure of oil tank.

1-29

Safety

Precautions for Maintenance

●      While disassembling connector or plug or disconnecting hose, stand on one side,

loosen slowly to gradually relieve internal pressure, and then take out.

●      The engine oil or oil plug will probably   spray out under the internal pressure of   traveling motor oil tank. Please slowly     loosen the oil plug to relieve the internal

pressure.

Guard against Splashed Fragments during Use of Hammer

During the use of hammer, the pins and the   metal fragments will probably fly out. It will probably cause serious personal harms.

●      While hammering hard metal parts such as pins, bucket teeth, side teeth, and bearings, please wear protective articles including

goggles and gloves.

●      While hammering pins and bucket teeth, ensure there is no person in surrounding

area.

Guard against High Pressure Grease

In the track pensioner, the grease is injected

under high pressure. If the tension is not

adjusted by the following designated procedure, the grease relief valve will probably fly out to     cause injuries.

●     Never loosen the grease pipe fittings.

●      Slowly loosen the grease relief valve. Do not rotate it for >1 turn.

●      Do not place your face, arms, legs, or body in front of the grease relief valve.

●      If no grease flows out when the grease

relief valve is loosened, the valve is

malfunctioned. Please contact a Ken stone Heavy Machinery service dealer for

repairs.

No Disassembling of Track Pensioner

The track pensioner is installed with a high

strength spring. If the track pensioner is

disassembled carelessly, the spring will spring

out to cause serious injuries. Do not disassemble the track pensioner.

1-30

Safety

Precautions for Maintenance

Operations of Additional Devices

Take special cautions while handling the high pressure nitrogen stored in the tank. The

improper handling will probably result in

explosion of nitrogen to cause serious injuries.

Please strictly abide by the following safety measures:

●      Do not break up or disassemble.

●      Keep away from open fire and fire source.

●     Never drill hole, weld, or melt.

●      Guard against physical vibration, such as hammering, rolling, and falling.

●      Bleed the sealed air before handling the cylinder body. If you need help, please  contact a Ken stone Heavy Machinery   service dealer.

Disconnection of Battery Cables

Please disconnect the battery cables before

operations or electric welding of electronic

system. Firstly disconnect the negative (-) cable

of battery. During re connection, finally connect the negative (-) cable of battery.

Caution during Handling of Battery

●      The battery contains sulfur acid. The

accidental touch will harm your eyes or skin.

In event of accidental contact with eyes, immediately flush by clean water and     quickly seek for medical treatment.

In event of accidental swallowing, drink  a lot of water or milk and immediately seek  for medical treatment.

If the sulfur acid comes into contact with your skin or clothing, immediately flush  by a lot of water.

●      Wear goggles and gloves during  handling of battery.

1-31

Safety

Precautions for Maintenance

| ● | The battery can generate inflammable hydrogen to probably cause explosion. Keep away from fire sources including open fire, spark, and ignited cigarette. | electrolyte will leak out. The contact with such electrolyte can harm your skin or corrode the machine parts. ● Clean the surrounding area of electrolyte |
| --- | --- | --- |
| ● | Use a flashlight to check the electrolyte level. | level marking by a wet cloth and check the level. Do not clean by a dry cloth, |
| ● | Ensure to turn off the starter switch and stop the engine before checking or handling battery. | otherwise it can cause electrostatic accumulation and combustion or Perliocslny. Replaced Critical Safety |
| ● | Never touch the electrodes by any metal tool or object, in order to prevent short- circuit. | Parts ● To guarantee the safe operation of machine for a longer time, add the oil and fulfill the |
| ● | The loose electrodes can generate electric spark. Ensure to tighten any loose electrode. | checking and maintenance periodically. To promote the safety, periodically replace the critical safety parts, including hoses and |
| ● | Ensure that the battery cover is securely covered. | seatbelts. To understand more details, please refer to the section “Periodically |
| ● | If the battery is frozen, do not charge the battery or attempt the jumper start of engine, otherwise it will probably cause explosion. Before use, heat the frozen battery to 15°C. | Replaced Critical Safety Parts” . ● “Periodically replaced critical safety parts” refers to the parts aged, worn, and functionally deteriorated after repeated use. The performances of such parts will |
| ● | Do not use the battery when the electrolyte level is below the lower limit. Otherwise it will speed up the internal aging and shorten the life of battery. In addition, it can lead to burst (explosion). | change along with time. These characteristics of such parts can cause serious mechanical damages or personal harms and the remaining lives of such parts can’t be judged by merely |
| ● | Do not fill the distilled water till the level is above the upper limit. Otherwise the | appearance checking or operation hand-feel. |

1-32

Safety

Precautions for Maintenance

●     Upon detection of any visible damage on the appearance, please replace the

“periodically replaced critical safety parts”, even if the designated replacement period    is not reached.

Jumper Start with Battery Charging Cable

●      To start the engine by battery charging

cable, ensure to connect the cable correctly as per following procedure. The incorrect   connection of cable can cause discharging   and battery explosion.

●      Keep the “Malfunctioned machine” and “Rescue machine” away from mutual     contact.

●     Never touch the positive (+) and negative (-) electrodes of battery charging cable

with each other or with the machine.

●      During connection, firstly connect the

positive electrode of battery charging cable to positive (+) terminal. During

disconnection, firstly disconnect the

negative (-) terminal (grounding terminal) from the negative cable.

●      Ensure to connect the cable clamps securely.

●      Connect the last clamp of battery charging cable to a place as far as possible from the battery.

●      To start the engine with battery charging

cable, always wear goggles and gloves.

●      Use the battery charging cable and clamps of dimensions suitable to the battery

capacity. Never use damaged or corroded battery charging cable or clamps.

●      Ensure that the battery capacity is same for the “Malfunctioned machine” and “Rescue machine” .

Asking Ken stone Heavy Machinery  Service Dealer for Welding Repair    The welding operations, if necessary, must be fulfilled by competent personnel in a working site with complete equipment. To prevent

damaging any machine part by over-high

current or electric spark, please abide by the following precautions.

●      Disconnect the battery cables before electric welding.

●      Do not apply 200V or higher voltage continuously.

●      Connect the grounding point within 1m reach from the welding area. Do not

connect the grounding terminal near the electronic control device/instrument or   connector.

●      Ensure there is no seal ring or bearing between welding area and grounding   terminal.

●      Do not connect the grounding terminal  to

1-33

Safety

Precautions for Maintenance

the any place near the working device pin or hydraulic cylinder.

●     Before the welding of machine body,   disconnect the connector of electronic control device.

Vibration Applied on Operator

According  to  the  results  of  the  test   on  the vibration transmitted from the machine to the

operator, the vibration applied on the operator’s

upper limbs is <2.5m/s2 and the vibration

applied  on  the  operator’s  body  in  the  seat is <0.5m/s2.

Checking after Maintenance

Accelerate the engine speed slowly from     low idling speed to fastest speed and check for leakage of oil or water from repaired

parts.

●      Operate all joysticks and check machine for normal operations.

Waste Disposal

●      Ensure to collect the drained oil  of

machine  into  a  container.  The  improper treatment  of  waste   oil   will  pollute   the environment.

●      While disposing harmful objects,

including lubricating oil, fuel, coolant,

solvent, filter, and battery, please abide by the applicable laws and regulations.

Disposal of Harmful Chemicals

The direct contact with harmful chemicals will cause serious harms. The harmful chemicals

used in this machine include oils/greases,

battery electrolyte, coolant, paint, and adhesive. Please dispose the harmful chemicals carefully  and properly.

1-34

Safety

Safety Signs

Safety Signs

To guarantee the safety of operator and operation personnel in the working zone, please set up the  following safety signs (markings) at some portions of machine. Walk around the machine with this manual and observe the contents and placement locations of these safety signs. Please review these signs and operation instructions contained in this manual jointly with the machine driver.

●      The safety signs shall be clean, clear, and easily legible. If any safety label falls, is damaged, or     becomes illegible, please replace with new one. Please provide your product serial number  while ordering new signs from a Ken stone Heavy Machinery service dealer.

●      If a part/body on which a safety sign is affixed is already replaced, affix a new sign to new part/body.

1-35

Safety

Safety Signs

1-36

Safety

Safety Signs

1. Warning

Carefully read and understand the manual   before operations, checking, and repairs of machine.

2. Safe distance.

Never approach to or stand in the working zone of machine.

3. Danger from underbody bolts thrown   from track roller adjuster. To guarantee  the safe and correct operations, ensure to read the operation manual before

adjusting the track roller adjuster.

4. Risk from rotating parts. Please turn off before checking and maintenances.

5. Safe distance and danger from

collision with working device. Please keep away from machine during

operations.

1-37

Safety

Safety Signs

6. This sign denotes not  to touch any hot part  during operations or  just after

operations.  Do not touch before  these parts, including engine,  pump, and exhaust  pipe,   are cooled  down.

7. This sign denotes that the entanglement   into  steering belt can  cause injury danger. Ensure that the  steering belt completely

stops rotation before  maintenance or

repairs.

8. This sign denotes that opening the  radiator or hydraulic oil tank  before its cools down  will

probably cause  scalding risk by the  spraying hot water  or oil. Cool down the  radiator or    hydraulic oil tank  before opening the  cap.

9.

Caution

Maximum lateral angle at 10°

Take special cautions while driving over a     slope or during operations on rugged road.    If the lateral angle is >10°, the machine will  roll over. During the operations of machine, always adjust the tracks to the outer

position. The inner position of the tracks is only used for driving through narrow

passages.

1-38

Safety

Safety Signs

10.

1.      The    operations      or     maintenance     of     this

machine    by    incorrect     method    will    cause bodily injury accident.

2.      Before  operating the machine, ensure to  read

and understand the  operation  manual  attached in the machine.

3.      Ensure  that  all safety  facilities  are  ready and

effective.

4.      Before  operating the  machine, please fasten

the seat belt.

Warning

11.

Warning

If the machine is installed with an enlarged  bucket, the cab will probably be impacted     when the boom is at the maximum  leftward deflection position.

12.

Caution

The hydraulic oil tank must contain a

certain pressure to prevent damaging the

water pump. Refer to the instructions on the operation manual.

13. Diesel

14. It’s prohibited to touch any joystick or

traveling joystick  while getting on or  off the     machine,  otherwise the parts  will be damaged.

Warning

15.  There is a danger of  electric shock during operations of cables.

1-39

Safety

Safety Signs

16. Hydraulic oil

17. Fixed point

18. Lifting direction

19. When the machine is lifted, fix it by

cushion blocks or supports. Do not  operate beneath the lifted machine  when the

machine is not fixed by any  support.

1-40

20. Keep the boom and bulldozing

blade  separated. While lowering the boom, take cautions not to touch the  boom  with bulldozing blade.

21 、No people are allowed to stand in the working area

Safety

Safety Signs

22 、Pay attention when relieving pressure

23 、Keep away from the engine fan

1-41

Controls

2-1

Control

Component Name

Component Name

| Up frame | Chassis | Working device |
| --- | --- | --- |
| 1 、seat | 5 、Track | 11 、Bucket |
| 2 、Engine hood | 6 、Drive wheel | 12 、Bucket cylinder |
| 3 、Hydraulic oil tank | 7 、Track roller | 13 、Arm |
| 4 、Fuel tank | 8 、Traveling motor | 14 、Arm cylinder |
|  | 9 、Bulldozer blade | 15 、Boom |
|  | 10 、Cylinder of blade | 16 、Boom cylinder |
|  |  | 17 、Swing joint |
|  |  | 18 、Swing cylinder |
|  |  | 19 、Hydraulic circuit |

2-2

Control

Component Name

1.      Left traveling lever

2.     Right travelinglever

3.       Bulldozer blade lever

4.      Lever for telescopic track

5.       Boom swing lever

6.       Right joystick

7.     Left joystick

8.     Safety lock handle

9.     Start switch

10.   Throttle lever

11.   Water temperature meter

12.   Warning sign

13.   Hour meter

14.     Lighting switch

15.     Switch auxiliary oil circuit button

The specific configuration depends on the machine specification or selected product.

2-3

Control Cover

Start Key

The start key is used to start the engine.

Repair cover (rear cover, hood, engine cover)

Warning

Before opening the repair cover, ensure that the safety lock handle is at locking position   and the engine is stopped.

When opening or closing the service cover,   please be careful not to pinch your hands or other parts of your body.

Opening

1. Insert the hood key and turn it

counterclockwise to unlock the maintenance cover.

2. Lift the hood lock and lift the maintenance

cover Closing

1. Unlock the limit lever and close the maintenance cover.

2. Insert the hood key and turn it clockwise to lock the maintenance cover.

2-4

Control Cover

Fuse Case

This case is functioned to protect the electric system against over-current.

Open

1. Insert the hood key and turn it

counterclockwise to unlock the maintenance cover.

2. Lift the hood lock and lift the maintenance cover.

3. Locate the fuse box. Close

1. Unlock the limit lever and close the maintenance cover.

2. Insert the hood key and turn it clockwise to lock the maintenance cover.

Fuel filler port

Warning

●     During the refueling into the fuel tank, keep  away  from  heat  source  and  open fire and do not smoke.

●     Refuel the fuel tank in a well-ventilated

place. Stop the engine before refueling.

●     Any overflown fuel shall be wiped away immediately.

●     Do not top up the fuel tank. Leave an expansion space for the fuel.

●      Tighten the fuel tank cap securely.

Opening

1. Insert the hood key and turn it

counterclockwise to unlock the maintenance cover.

2. Lift the hood lock and lift the maintenance cover.

.3. Hold the fuel tank cap and turn it

counterclockwise to unlock the fuel cap.

Closing

Fasten the fuel tank cap and turn it clockwise to lock the fuel cap.

2-5

Control Seat

Seat

|  | Warning |  |
| --- | --- | --- |

Adjustment and Fixing of Seats (For high-end seats only)

A. Adjust the backrest angle

1. Stand up straight and sit back on the seat.

2.  Adjust  the backrest  to  the  angle  you  want by rotating the handle (1).

B. Front and back adjustment

1.  Pull  up  the  adjustment  lever  (2), slide  the seat back and forth, and adjust it to the position you want when operating the machine.

2.  Loosen  the  adjustment  handle  (2)  at  the position you want to fix the seat.

2-6

Records

2-7

Control

Electrical control panel

Panel Name and Description

| no. | name | note |
| --- | --- | --- |
| 1 | hour meter | Display working hours, the range is 0-9999.9h. |
| 2 | Charging warning light | When the engine is running, if there is a problem with the charging system, the light flashes and an alarm sounds. |
|  | Oil pressure indicator | When the engine is running, if the oil pressure is abnormally low, the light flashes and an alarm sounds. |
|  | Work light indicator | Show the working conditions of the work lights. |
|  | Warm-up indicator | When the engine is warmed up, the light goes out. |
| 3 | Water temperature meter | Shows the change of cooling water temperature. |

2-8

Control

Electrical control panel

Precautions

This instrument is a water-proof instrument, featuring stable performance and high

conciseness and elegance, and is an

intelligent instrument developed specially for small excavators. To guarantee the stable

data and prolong the life of instrument, please notice the following issues:

1. Do not disassemble the instrument without permission.

2. Never brutally hammer or knock the instrument.

3. Guard against external damages.

4. Keep clean the panel. Any organic solvent  or corrosive liquid, including diesel and   gasoline,   shall be timely wiped away, in  order to prevent  corroding the panel.

5. Should you have any question during use, please timely contact service personnel.

After-Service Information

Provided that the operations are performed

normally as per the instructions of the manual,  the malfunctions occurred within the warranty  period will be repaired free of charge. Please     carry this warranty certificate during the repair of this product and repair by a sales point  of     this device. The repair expense will be charged under the following conditions:

1. No warranty certificate.

2. Malfunction due to incorrect use of this

device or non-authorized repair or  modification of this device.

3. Malfunction due to falling at user’s default during transport after purchase.

4. Malfunction due to fire, earthquake, gas

leakage, windstorm, thunder and lightning,  and abnormal voltage.

5. The purchase date or customer name is not recorded in this manual or the information  is altered.

2-9

Control

Switches

Start Switch

Horn button

Important: Do not rotate the starter key

from OFF to ON and then from ON to OFF repeatedly within a short time, otherwise it   will probably result in engine malfunction.

OFF …… This position is used to stop the

engine and insert and withdraw the starter key. ON …… Engine running position. All electric devices are functional at this position.

START …… Engine running position. Upon     release of key, the switch automatically returns to ON.

If the coolant temperature is too low,

rotate  counter-clockwise the key, hold for 10~15s, and  then release before start.

Rotate the key to ON  and then to START to start the engine.

2-10

Control

Joysticks

Joysticks

Safety lock handle

|  | ΔWarning |  |
| --- | --- | --- |

●      Before standing up from the driver seat  to adjust the operator’s seat, lower the    working device to the ground, lift up the safety lock handle to lock it, and stop

the engine. When the safety lock handle is lowered down (Unlocked), the

accidental touch of any joystick will

result in sudden movement of machine and cause serious injuries or deaths.

●      Please be noted that, even if the safety

lock handle is placed at locking position, the bulldozing blade, boom, and

auxiliary hydraulic controls can’t be locked.

Do not touch such controls accidentally.

●     While lifting up or lowering down the   safety lock handle, take cautions not to touch any joystick.

●      Before leaving the driver seat, lower the working device onto the ground, lift up   the safety lock handle to locking

position, and stop the engine.

Meanwhile, ensure to withdraw the key,  close the doors and hoods, and carry the key with you and then preserve it in a

designated place.

This device is functioned to lock the operations of bucket working device and machine slewing.

When pushing the lever forward, except for the walking lever, the other levers cannot work

Accelerator joystick

Used to control engine speed. (A)...low idle

(B)...Maximum speed Joysticks

ΔWarning

●      Before starting operations, please carefully check and confirm the joystick mode to be used.

2-11

Control

Joysticks

●      The descriptions in this manual use the ISO mode of machine.

|  | ΔWarning |  |
| --- | --- | --- |

Control Mode

Bulldozing blade joystick

Use this joystick to operate the bulldozing blade.

(A) … Lower the bulldozing blade. (B) … .Lift the bulldozing blade.

Traveling Joystick

●      Before operating the traveling

joystick, ensure that the  bulldozing

blade is in front of the driver  seat.

Please bear in mind that the  operation  direction of the traveling joystick with  bulldozing blade  behind the driver seat is reverse to that  with bulldozing blade in front of driver  seat.

Use these two levers to move forward or backward and change direction.

2-12

Control

Joysticks

Boom swing lever

This lever controls the hydraulic oil flow and direction in the auxiliary hydraulic circuit.

(A)  ......Boom swing right (B) ......Boom swing left

Auxiliary hydraulic circuit control button

This button is used for the operation of

attachments that require hydraulic power such as breakers.

1. First of all, make sure that you have correctly  installed hydraulic attachments such as breakers. 2. Keep pressing this button to control the

hydraulic attachments, and automatically return to normal control after releasing it.

2-13

Control

Accessory

Auxiliary hydraulic pipeline (If installed)

|  | ΔWarning |  |
| --- | --- | --- |

Disconnecting the pipelines before the

pressure relief of hydraulic system will

probably result in out-spray of hydraulic oil.

●     After the stop of engine, immediately    press auxiliary switch for several times to  relieve the pressure in the auxiliary hydraulic circuit.

●      Slowly loosen the bleeding plug to

relieve the internal pressure of  hydraulic oil tank.

●     While disconnecting hoses, stand on one  side and loosen slowly. Slowly relieve the internal pressure before disassembling.

These pipelines transport the hydraulic oil needed to operate hydraulic breakers or

other working devices.

(1)... Auxiliary hydraulic pipeline

Shut-off valve (S): Close

(0): open

2-14

Control

Accessory

Connection of Hydraulic Circuits

To connect the attachment hydraulic pipeline, please operate as per following procedure:

1.      Relieve the residual pressure from system

and then close the shutoff valve.

2.     Take out the plug.

3.      Connect the attachment hydraulic pipes to

ports (a). While installing a  hydraulic

breaking hammer, connect the  oil inlet port

to port (a) and oil return port  to port (b).

4.      Open the shutoff valve. While installing a

hydraulic breaking hammer, open the selector valve (1).

5.      After connections, fully bleed the air from

hydraulic pipes.

a.       Start the engine and run at low idling

speed under no-load condition for

10min.

b.      During the running of engine at low idling speed, operate the hydraulic   auxiliary switch repeatedly (for

approximately 10 times) to bleed the air from hydraulic pipes.

c.       Stop the engine and wait for at least 5min, till the air bubbles overflow

from the hydraulic oil in the oil tank.

Important: If applicable, follow the  attachment manufacturer’s procedure to  bleed the air.

6. Check for presence of oil leakage.

Disconnection of Hydraulic Circuits

1.      Relieve  the  residual  pressure  from  system

and then close the shutoff valve.

2.      Disconnect the pipelines from ports.

3.      Install the plug.

Relieve residual pressure

After using the auxiliary hydraulic circuit,    there is residual pressure in the circuit. This

is called residual pressure. Before

disconnecting the pipeline, release the residual pressure.

2-15

Operations

3-1

Operations

Before Start of Operations

Before Operations

Getting on/off Machine

ΔWarning

●      Do not jump on or off the machine. Do not attempt to get on or off a moving machine.

●     While getting on/off the machine, hold  the handrails to support your body weight  and maintain three-point balance posture

(Hands and feet) for your body.

●      Do not use safety lock handle or any joystick as handrail.

Walk-Around Checking

Before the first start of engine each day, fulfill one walk-around checking.

Refer to page 5-14 and 5-15 “Maintenance, Walk-Around Checking” .

Daily Routine Checking

Before the first start of engine each day, fulfill one walk-around checking.

Refer to page from 5-17 to 5-21 “Maintenance, daily routine inspection” .

3-2

Operations

Start and Stop of Engine

Start and Stop of Engine Before Start of Engine

1.         Adjust the seat to comfortable operating

position.

2.     Check and ensure that the safety lock

handle is at locking position.

3.     Check and ensure that all joysticks are in

neutral position.

4.     Insert the key into start switch, rotate to

ON position, and then check as below:

●      All warning lamps turn on for 1s and the warning tone is issued for 2s. The

instruments start working.

●      Press the lamp switch and check and ensure that the boom lamp turn on.

If a lamp fail to turn on or a warning lamp fails to sound, the lamp bulb or wire is probably

damaged. Please contact Kenstone service agent

for repair.

3-3

Operations

Start and Stop of Engine

Start of Engine

|  | Warning |  |
| --- | --- | --- |

●      Keep all personnel away from working

zone.

●      Sound the horn to alert the personnel around machine.

Important: Do not run the starter motor

continuously for >15s. If the engine fails to

start, wait for 60s and then attempt to restart  the engine.

Important: If the engine stops due to  shortage   of fuel, please add fuel, rotate the  key to ON for 60s, and then rotate it to  START position.

Running the starter motor  for a long time before the supply of sufficient  fuel will cause start

failure of starter motor.

Normal Start

1.     Pull the accelerator joystick to neutral

position(B).

2.     Rotate the starter key to START position

to start the engine.

3.     After the engine is started, release the key.

The key will automatically return to ON position.

4.     Check and ensure that all warning lamps

are already off.

|  | ΔWarning |  |
| --- | --- | --- |

5.     Return the accelerator joystick Start in Cold Weathers

Never use starting fluid on this engine, as  the starting fluid will probably explode.

1.     Pull the accelerator joystick to neutral

position(B).

3-4

Operations

Start and Stop of Engine

2.      Rotate the start key counter-clockwise to     HEAT position for approximately 10s. The preheated indicator lamp turns on   to

indicate the running of preheated.

3.      Rotate the key to ON position, wait for the  instrument lamp to turn off and the buzzer   to stop sounding, and then rotate the key to START.

4.      After the engine is started, release the key.

The key will automatically return to ON position.

5.      Check and ensure that all warning lamps

are already off.

6.      Return the accelerator joystick and warm

up the engine. Refer to “Warm up the engine "  item

Warm up the engine

Important: Before the completion of warm-up. Avoid the full-speed running of engine. Do not   warm up the engine for a long time (20min or    longer). When the idling is required,

occasionally apply the load or run the engine at medium speed.

Return   the   accelerator  joystick   and   then run  the  engine  at  low  idling  speed  under no-load condition for 5min.

3-5

Operations

Start and Stop of Engine

Stop of Engine

· Important: Do not stop the engine suddenly     · under heavy load or during high-speed  running. Otherwise it will result probably in  overheating or blockage of engine. Do not   stop the engine    hastily, unless in event of emergency

1. Return to the throttle lever.

2. Let the engine idle for about 5 minutes.

3. Turn the starter key to the OFF position to turn off the engine.

3-6

Operations

Operate Machine

Operatie Machine

Joystick Mode (ISO Mode)

ΔWarning

●      Before starting operations, please carefully check and confirm the joystick mode to be used.

●      The descriptions in this manual use the ISO mode of machine.

（ISO）

|  | Drive leftward |  | Drive rightward |  | Safety lock locked |
| --- | --- | --- | --- | --- | --- |
|  | Back up to left |  | Back up to right |  | Safety lock unlocked |
|  | Extend arm |  | Lower boom |  | Swing leftward the boom |
|  | Retract arm |  | Lift boom |  | Swing rightward the boom |
|  | Slew leftward upper frame |  | Load bucket |  | Lift bulldozing blade |
|  | Slew rightward upper frame |  | Dump bucket |  | Lower bulldozing blade |

3-7

Operations

Operate Machine

Warm up the engine (hydraulic oil)

| Warning |  |
| --- | --- |

It’s really dangerous to operate the working device without warm-up (Hydraulic oil), as  the working device can’t respond quickly to the controls or will motion probably in

unexpected manner and the safety devices    can’t work normally probably. Therefore,     ensure to warm up the machine sufficiently.

Important: When the hydraulic oil

temperature is <20℃, do not operate any    joystick quickly. During the operations, the appropriate hydraulic oil temperature is

50~80℃. If it’s necessary to operate under   low temperature, warm up the hydraulic oil to at least 20℃.

Normal Warm-up

speed under no-load condition for approximately 5min.

2. Completely lower down the safety lock

handle to unlock and lift the bucket from the ground.

3.      Slowly telescope all cylinders for several

times under no-load condition.

4.      Slew slowly leftward and rightward for

several times.

1.      Pull the accelerator joystick to neutral

position. Then, run the  engine  at  medium

3-8

Operations

Operate Machine

5.     Drive forward and backward slowly for

several times.

Warm-up in Cold Weathers

1.     Fulfill the normal warm-up procedure.

2.     Operate the bucket cylinder to an end and

hold.

Do not hold at this position more than 30s.

3.     Repeat step 2, till the running speed of the

bucket is normal.

Checking after Warm-up

After the warm-up of engine and machine (Hydraulic oil), check as per following

procedure and when necessary repair.

1.     Check the warning lamps and instruments

as below:

●      Whether all warning lamps turn off.

●      Whether the water temperature meter is ≤105℃ .

2. Check for normal engine exhaust gas color,

sound, and vibration.

3.  Lift  up  the   safety  lock  handle  to  locking

position  and  check  whether  the  operation and traveling joysticks are locked.

3-9

Operations

Operate Machine

Change of Chassis Width

|  | Warning |  |
| --- | --- | --- |

Operating Machine with Maximum Width

●     Always operate the machine with

extended chassis width of 950mm, in

order to improve the machine stability to the maximum extent. Operating the machine with narrow chassis width

(800mm) will probably result in rollover of machine due to poor stability.

●     If it’s necessary to operate the machine with narrow chassis width (800mm),

retract the bucket working device and lower the boom to reduce the gravity   center and face the  machine towards   front before traveling.

This machine can change the track width.

Normally, to keep stable of machine, it’s better to operate with extended track width.

Change of chassis Width

Operate in a level and solid ground without obstacles.

1. Lift the machine body by bucket working

device and bulldozing blade.

Important: Ensure to lift the machine body   before the change of track width. Attempting the operation with tracks on ground will

probably damage the side traveling frame or span cylinder.

2.     Unlock the safety lever.

3-10

Operations

Operate Machine

3. Push the lever forward to increase the width of the chassis to 950 mm.

4. Pull the lever backward to reduce the width of the chassis to 800 mm.

Change of bulldozer blade width

1. loosen the bolts(1) and take out the

pins(2)

2.      Rotate the extension plate around the pins

for 180°.

3. Install the pins and tighten the bolts.

3-11

Operations

Operate Machine

Operations of Traveling Joystick

|  | ΔWarning |  |
| --- | --- | --- |

●     Do not allow any person to access the turning radius or path of machine.

●      There are blind zones behind the

machine. To reverse the machine, if necessary, slew the cab to check the  safety and ensure there is no person behind the machine.

●      Before operating the traveling

joystick, ensure that the  bulldozing

blade is in front of the driver  seat.

Please bear in mind that the  operation  direction of the traveling joystick with  bulldozing blade  behind the driver seat is reverse to that  with bulldozing blade in front of driver  seat.

●     Remove all obstacles from the path of machine.

Move the machine back and forth

1.     Pull the accelerator joystick to increase the

engine speed.

1.     Fully lower the safety lock lever to release

the lock.

3-12

Operations

Operate Machine

3.      Retract the bucket and   lower to 30~40cm   ·

off the ground.

4.      Lift the bulldozing blade.

5.      Operate the traveling joystick as per

following procedure.

When the bulldozing blade is in the front of seat:

To drive forward: Pull forward the joystick. To drive backward: Pull backward the

joystick.

When the bulldozing blade is behind of seat: To drive forward: Pull backward the

joystick.

To drive backward: Pull forward the joystick.

Pivot Steering

Turn left when parking:

to turn forward left: push the right traveling lever;

to turn back left: pull the right traveling lever.

Turn right when parking, same operating to operate the left traveling lever.

3-13

Operations

Operate Machine

Turn left during traveling:

To turn left during traveling forward:

Place the left joystick to neutral position. To turn left when traveling backward:

Place the left joystick to neutral position.

To turn right during traveling, operate the  right  traveling lever by same operation method of left traveling lever.

Rotate in situ

Rotate left in situ: pull left traveling lever    backward and push the right traveling lever forward.

Rotate right in situ: pull right traveling

lever backward and push the left traveling lever forward.

Parking

|  | Δ Warning |  |
| --- | --- | --- |

●      Park the machine on a level, solid, and  safe ground. Set the parking device. If   it's necessary to park the machine on a slope, block the track by wedges to

prevent movement of machine.

●     When the safety lock handle is unlocked, the accidental touch of any joystick will   result in sudden movement of machine

and cause serious injuries or deaths.

●      Please be noted that, even if the safety

lock handle is placed at locking position, the bulldozing blade, boom, and

auxiliary hydraulic controls can’t be locked.

Do not touch such controls accidentally

|  | Caution |  |
| --- | --- | --- |

Do not stop the machine hastily, unless in

event of emergency.  Stop the machine at the optimal timing whenever possible.

3-14

Operations

Operate Machine

Slowly place the left and right traveling  levers to neutral position. Stop the  machine.

Operations of Working Device

|  | ΔWarning |  |
| --- | --- | --- |

●      Before starting operations, please

carefully check and confirm the joystick mode to be used.

●      The descriptions in this manual use the ISO mode of machine.

Use the right joystick to control the boom and bucket.

Use the left joystick to operate the arm and slewing.

Return the joysticks to neutral position to stop the bucket working device.

Lower down the safety lock handle to the unlocking position.

Slewing

|  | Warning |  |
| --- | --- | --- |

Before slewing, check the surrounding area for safety.

Slew the up-frame left:  Push  the left joystick left.

Slew the up-frame right:  Push the left joystick right.

Operate the boom

Lower boom: Push the right joystick forward .

Raise the boom: Push the right joystick backward .

3-15

Operations

Operate Machine

Swing of Boom

Operations of Arm

Retract bucket arm: Push the left joystick backward.

Extend bucket arm: Push the left joystick forward.

Operations of Bucket

Load bucket: Push the right joystick left.

Dump bucket: Push the right joystick right.

Swing the boom left:  push this lever forward.

Swing the boom right:  pull this lever backward.

Operations of Bulldozing Blade

Lower bulldozing blade:  Push this lever forward.

Raise bulldozing blade:  Pull this lever backward.

3-16

Operations

Operations Procedures

Operations Procedures

Prohibited Operations

|  | ΔWarning |  |
| --- | --- | --- |

●     Do not operate on a bed rock (hard or soft).

●     Do not slew during the traveling. If it’s    necessary to operate the bucket working device during traveling, operate at

sufficiently low speed to ensure the complete control at all times.

Do not use the slewing force for

demolition  or leveling operations.

Do not use slewing force to dismantle walls or level ground. In addition, do not insert the

bucket teeth into the ground during slewing.  Otherwise the bucket working device will be damaged.

No Digging during Traveling

Do not insert the bucket into the ground or use the traveling force for digging.

Steady Operations of Hydraulic Cylinders

Do not extend any hydraulic cylinder to the end. Reserve an allowance during operations.

3-17

Operations

Operations Procedures

When  the  machine  body  is   lowered   and   the bucket  arm cylinder is  completely extended, do not    support    the   machine    body   by   bucket working   device.   Otherwise,   the   load   will   be concentrated   on  the   bucket    arm   cylinder    to probably damage the bucket arm cylinder.

No Digging by Pulling or Hammering Bucket

This will shorten the life of bucket working device. Dig by hydraulic force.

Do not operate by the down force of machine.

Using  additional  tension  onto  the  machine  will shorten the life of machine. During digging, use the   hydraulic   force   of   cylinders   by    shallow depth and large travel.

Digging of BedRock

For hard bed rocks, it’s preferable to break the

3-18

Operations

Operations Procedures

rock into small pieces by breaking hammer     before digging. It can prevent the damages of machine and thus can be more economic.

Guard Bulldozing Blade against

Impact

Impacting the bulldozing blade with rock will damage the bulldozing blade and bulldozing   blade cylinder.

Careful Retraction of Bucket Working Device

While retracting the bucket working device, take cautions not to impact the bucket with   bulldozing blade.

Prohibited Use of Bulldozing Blade as Support

Caution of Bulldozing Blade during Digging

For deep digging operations, if the bulldozing blade is in the front, take cautions to prevent

impacting the boom and bucket with bulldozing blade.

During operations, place the bulldozing blade on the rear of machine whenever possible.

3-19

Operations

Operations Procedures

Caution of Downward Digging with Bulldozing Blade

This bulldozing blade is designed for simple bulldozing operations. Do not use the

bulldozing blade for deep digging. Otherwise,

the bulldozing blade and the under frame will probably be damaged.

Precautions for Operations Precautions during Traveling

Driving the machine over obstacles (Including rocks and stumps) will probably apply high

load onto the machine body and damage the machine. Avoid driving over obstacles

whenever possible. If it’s necessary to drive

over obstacles, close the bucket working device to the ground, drive the machine at low speed,   and drive the machine in such manner that the    obstacles are in the middle of tracks.

Precautions for Operations of This Machine in Water

It's prohibited to immerse the rear of  machine in the water.

3-20

Operations

Operations Procedures

|  | ΔWarning |  |
| --- | --- | --- |

●      Permissible water depth

Operate this machine in the water only when the water level is not above the

·       middle of track shoe (1).                               ·

●      For  those parts  used  in water  for  a  long

time, add sufficient grease till the used grease is squeezed out.

●      Do not immerse the slewing support or body in the water or sand. If already

immersed, please contact a Ken stone service dealer for checking.

Precautions for Traveling on Slopes

●     Do not drive the machine on a slope

steep enough to cause instability of

machine. Please be noted that, in the

actual applications, the performances of machine on a slope will be deteriorated   due to severe working conditions.

●     While traveling on a slope, lower the

bucket to 20~30cm off the ground.

While climbing up a steep slope, extend the bucket working device to the front   position. In event of an emergency,

lower the bucket onto the ground and stop the engine of machine.

●     While traveling on a slope, drive slowly in 1st gear (Low speed).

●      Keep the driver seat facing towards the up slope direction while climbing up a

slope. Keep the driver seat facing

towards the down slope direction during down slope traveling. In both cases, pay  special attention to the ground ahead of

the machine during traveling.

●     Do not reverse the machine down a slope.

3-21

Operations

Operations Procedures

●     Do not change direction on a slope or

lateral slope. Firstly return the machine   to a level ground and then select another path.

●      The machine will probably slip laterally  while traveling on a gentle slope covered by grass or dry leaves or traveling on a    wet metal plate or frozen surface. Do

not park the machine laterally on a slope.

Traveling Posture on Slopes Up slope Traveling

To climb up a 15° or higher slope, maintain the machine posture as shown in above diagram.

Downs lope Traveling

While driving down a 15° or higher slope, reduce the engine speed and maintain the   machine posture shown in above diagram.

3-22

Operations

Operations Procedures

Braking during Downs lope Traveling

While driving down a slope, the brake  applies     braking automatically after the traveling joystick returns to neutral position.

In Event of Track Slip

In event of traveling failure due to track slip

while climbing up a slope, use the digging force of bucket arm to climb up the slope.

In Event of Engine Flame out

If the  engine  stops while driving down a slope, shift  the  traveling  joystick  to  neutral  position, stop the machine, and then restart the engine.

3-23

Operations

Operations Procedures

Out of the mud

If the  machine  is  entrapped  in the  mud,  drive ·  out as per following procedure.

If a track gets stuck in the mud

1.      Swing the bucket towards the track

entrapped in the mud.

2.      Set the angles at 90~110° for bucket arm

and boom.

3.      Push the bucket bottom (Not bucket teeth)

onto the ground.

4.      Place a wood plate or similar object

beneath the lifted track.

5.      Lift the bucket and drive out the machine

from the mud slowly.

If both tracks are stuck in the mud

1.    Operate above-mentioned steps 1~4 for

both tracks.

2.      Insert the bucket into the ground in the

front of machine.

3.      Pull the bucket joystick during forward

traveling to slowly drive out the machine.

Allowed Operations of Machine Digging

1.      Place the bulldozing blade to the opposite

side of the digging zone.

2.      Digby bucket arm and bucket. It’s

preferable to dig by shallow depth and

large travel. The maximum digging force   is achieved when the angle between boom and bucket arm is 80~120°. It’s preferable to change the angle for effective digging.

3-24

Operations

Operations Procedures

Ditching

Install a bucket suitable for digging of ditches, adjust the tracks to be parallel with the ditch to be dug, in order to improve the working

efficiency. While digging a wide ditch, dig the side faces and then the middle area.

Digging of Side Drainage Ditch

As shown in the diagram, dig the side ditch by swing function of boom.

Loading

While loading soil into the cargo body of a

truck, loading from the rear end of truck can load more soil easier than loading from the

front end. In addition, the small angle slewing can be used to improve the efficiency.

Leveling

1.      Close the bucket working device to the

machine body.

2.      Slowly push away the soil from the side

face of soil pile.

3.      If the soil pile is low, push the soil from

the top. If the load is too heavy for the

machine body, lift or lower the bulldozing blade for adjustment.

3-25

Operations

Parking of This Machine

Parking of This Machine Parking

ΔWarning

●      Stop the machine on a level, solid, and safe ground. Set the parking device.

Digging

Stopper

●     If it’s necessary to park the machine or tilt the machine on a slope, securely

park the machine and block the machine against movement.

●     While parking the machine in a street,   use gratings, warning signs, or lamps to keep the visible even in the night, in

order to  prevent  the impact  with other

vehicles.

●      Before leaving the seat, lift up the safety lock handle to the locking position and   stop the engine. Meanwhile, ensure to     withdraw the key, close the doors and

hoods, and carry the key with you and then preserve it in a designated place.

1.     Place the left and right traveling joysticks

to neutral positions.

2.     Push backward the accelerator joystick to

idle the engine.

3.     Lower the bucket and bulldozing blade

onto the ground.

4.     Lift up the safety lock handle to the

locking position.

For details, please refer to "Stopping the Engine" on page 3-6.

Checking after Stop of Engine

1.     Check  for  leakage  of  oil  and  water  and

check   the   working   device,   cover,   and under   frame.   Upon   detection    of   any abnormality, repair.

2.     Fully fill fuel.

Refer  to  page   5-21   “Checking   of  fuel level” .

3.     Remove  paper  fragments  and  dusts  from

engine compartment.

4.     Remove the mud from under frame.

3-26

Operations

Parking of This Machine

Locking

Ensure to lock the following parts.

3-27

Operations

Treatment in Cold Weathers

Treatment in Cold Weathers

Preparations for Cold Weathers

It’s difficult to start the engine in cold weathers. Replacement of Fuel and Lubricants

Replace the hydraulic oil, engine oil, and fuel with trademarks suitable to cold weathers.

Refer to page 5-4 “Fuel and Lubricant Table” .

Battery

The battery performance deteriorates along with the reduction of temperature.

Check the battery. Upon detection of battery

discharging, please contact a Ken stone service provider for charging.

Refer to page 5-28 Checking of battery

electrolyte level and adding of electrolyte” .

Precautions after Operation

Please  abide  by  the  following  precautions  to prevent the movement failure of machine due to frozen mud, water, or under frame.

●      Remove all mud and water from machine   body. Especially, wipe clean the hydraulic  cylinder rod to prevent the ingress of rod    surface mud and dusts into the seals along  with water drops from damaging the seals.

●      Park the machine on a solid and dry

ground. If impossible, park the machine on a ground paved with wood boards.

●      Fully drain the water from fuel tank to prevent freezing.

Refer to page 5-27 “Water drainage from fuel tank ”.

●      As the battery capacity will drop

remarkably in low  temperature

environment, cover the battery or take the  battery out of the machine and store it in a warm place. If the electrolyte level is too   low, add distilled water before starting

operations in the morning. To prevent the    freezing of battery electrolyte at nighttime, do not add water after the daytime

operations.

After Cold Weathers

When the weather becomes warm, fulfill the following operations.

●      Replace the fuel and lubricating oil for all parts as per the trademarks specified by     “Fuel and Lubricant Table” .

Refer to page 5-4 “Fuel and Lubricant

Table” .

3-28

Operations

Handling of Rubber Tracks

Handling of Rubber Tracks

This machine is equipped with rubber tracks and the rubber tracks have their intrinsic

weakness, namely low strength. Please ensure to abide by the following prohibitions and

precautions to prevent damage and falloff of rubber tracks.

Prohibition

Do not drive or operate the machine in following sites:

●      Driving and steering the machine  on

gravels, rough and hard rocks, steel beams, or scrap irons or near the edges of steel

plates will damage the rubber tracks.

●      Driving the machine on a riverbed or place · with numerous pebbles can probably result   in inclusion of pebbles in the tracks and

the damage or falloff of tracks.

●      Do not operate this machine by the sea.     The salt content will probably corrode the steel core.

●      Keep the tracks away from the adhesion of fuel, lubricating oil, salt, and chemical

solvent. These substances will probably  corrode the welds of track steel cores to  cause rusting or falloff. Such substances adhered onto the tracks shall be

immediately removed by water.

3-29

Operations

Handling of Rubber Tracks

●      Driving the machine on irregular surfaces, such as new asphalt pavement and hot

steel plates exposed to campfire or burning sun will result in irregular wear or damage  of tracks.

●      Do not move the rubber tracks to  possibly slip ground. Otherwise, the wear of the

tracks will be probably accelerated.

Precautions

Please abide by the following precautions during the operations of this machine:

●      Do not slew the under frame (The

superstructure is not slewed) when the

bucket working device is lifted in the front of machine body. Otherwise, the load will  be concentrated at single point of tracks

and the tracks will be twisted to cause rapid damage.

●      In all cases, avoid changing the route

suddenly or avoid the in-situ steering on concrete surface whenever possible.

Otherwise the rubber tracks will be worn or damaged.

●      Avoid the strong impact onto the rubber tracks due to falloff.

●      The tracks will be damaged by salt,

potassium chloride, ammonium  sulfate,

3-30

Operations

Handling of Rubber Tracks

potassium sulfate, and lime triple  super    phosphate. Such substances adhered  onto the tracks shall be immediately  removed  thoroughly by water.

●      Keep both sides of rubber tracks away

from friction with concrete or wall surface.

●      Do not impact the bucket with the rubber tracks, otherwise the tracks will be

damaged.

●      Take special cautions on snowy or icy surfaces in winter, as the tracks are

vulnerable to slip on such conditions.

●      Please use rubber tracks within -25℃~55℃ .

●      For long-time storage (≥3 months) of   rubber tracks, please store in an indoor place without direct sun exposure or

rainfall.

●      The rubber tracks are not as stable as steel tracks,  as  they  are  completely  made  of rubber.  Take  special  cautions  during  the lateral slewing and swing.

Guard against Falloff of Rubber Tracks

Abide by the following precautions to prevent the falloff of tracks:

●      Always maintain an appropriate tension for the tracks.

●      When  driving  over pebbles  or  large  steps (≥20cm  in  depth,  such  as  large  rocks), climb  on  the  steps  in right  angle  and  do not change route on steps.

●      While reversing up a slope, do not change direction at the start point of slope.

3-31

Operations

Handling of Rubber Tracks

●      Please avoid driving the machine with one  track on a slope or protrusion and the other track on level surface (In such case, this

machine is inclined by 10° or more).

Please drive the machine with both tracks on level surface.

●      Do not change the direction when the

tracks are loose as shown in the diagram.

●      In such case, the rubber tracks will fall off if the machine drives backward.

●      In such case, the rubber tracks will fall off if the machine slews.

3-32

Transportation

4-1

Transportation

Loading and Unloading

Loading and Unloading

|  | ΔWarning |  |
| --- | --- | --- |

The machine will probably roll over or fall off during unloading. Ensure to take the     following safety measures:

●      Select a solid and level ground and keep a sufficient spacing from the road curb.

●      Fix the ramps of sufficient strength and dimensions to the cargo body of truck.   The inclination of the ramps shall not     exceed 15°. If the ramps deflect

downward excessively, please support the ramps by supports or cushion

blocks.

●     Do not load or unload the machine by working device. Otherwise, it will

probably result in rollover or falloff of machine.

●      Keep the truck cargo body and the

ramps clean without oil, sand, ice/snow, or other impurity, in order to prevent     the side slip of machine. Clean the

tracks.

●      Block the wheels of transport truck by wedges to prevent movement.

●     While loading or unloading the machine, drive the machine slowly in 1st gear (low  speed) as per the signals of the signaler.

●     Do not change direction on ramps.

●     Do not slew/swing on ramps. Otherwise the machine will probably roll over.

●      Slewing/swinging the machine on the

cargo body of truck will probably result in unstable legs of machine. Therefore,   operate slowly.

●     If possible, lock the cab doors after    loading. Otherwise the cab doors will probably open during the transport.

●      Plug securely the tracks by wedges and then fix the machine to the truck cargo body securely by ropes or chains.

During the loading or unloading of machine, please ensure to use ramps or platform and    abide by following procedures.

Fix to suspension parts

Distance between

Stop block

15°or less

Ramp

1.     Securely apply the parking device of

transport truck and block the wheels by wedges.

2.     Place the ramps securely onto the truck

cargo body. The inclination of the ramps shall not exceed 15°.

4-2

Transportation

Loading and Unloading

3.      Align  the  center  of truck  cargo  body  with

the  center  of machine  and  align  the  center of ramps with center of tracks.

4.      Ensure  that the  bulldozing blade  will not

touch the ramps.

5.      Lower  the bucket  working  device  as  low

as possible and take cautions not to  impact it with the transport truck.

6.      Reduce engine speed.

7.      According  to  the  signaler’s  signals,  drive

the  machine   straightly  up  or  down   along the ramps in  1st gear (low speed).

8.      Load  the machine to designated position of

transport truck. Refer to “Transport Status” in page 4-6 for details.

4-3

Transportation

Lifting of Machine

Lifting of Machine

|  | ΔWarning |  |
| --- | --- | --- |

●     Master and use the correct lifting signals.

●     Daily check the lifting device for

damaged and missing parts and when necessary replace.

●      Please use the ropes of sufficient

capacity for the machine weight during lifting.

●     Lift the machine as per the procedure

described below. Do not operate by any   other method. This is really dangerous    as it will probably result in imbalance of machine.

●     Do not operate the lifting if there is any operator on the machine.

●      Operate slowly during lifting to prevent the rollover of machine.

●     During lifting, keep all personnel away from working zone. Do not move the     machine over any person.

Important: This lifting method is applicable for the models with standard overall

parameters. The gravity center varies  depending on the installed attachments and  optional

devices.

To understand more details, please contact  your Ken stone service dealer.

Lifting

1.     Slew the superstructure to place the

bulldozing blade behind the machine     (Adjust the superstructure to be parallel with track frame).

2.     Lift the bulldozing blade to highest

position.

3.     Sufficiently extend the bucket cylinder and

bucket arm cylinder and lift the boom to highest position.

4.     If the boom swings leftward or rightward,

adjust it to neutral position.

5.     Raise the safety lock control lever to the

locking position.

6.     Stop the engine, withdraw the starter key,

and leave the machine.

7.     Install the ropes as shown in diagram

below. Install the ropes and lifting

appliance and take caution to keep them away from machine body.

8.     Slowly lift the machine, till it's off the

ground.

9.     Stop the lifting till the machine becomes

stable and then lift the machine again.

4-4

Transportation

Lifting of Machine

unit: mm

4-5

Transportation

Fixing of Machine

Fixing of Machine

After loading the machine to designated position, fix as per following requirements. Transport Status

1.     Lower the bulldozing blade.

2.     Sufficiently extend the bucket cylinder and

bucket arm cylinder and then lower the boom.

3.     Lift up the safety lock handle to the

locking position.

4.     Stop the engine, withdraw the starter key,

and lock all locks.

5.     Place the stop blocks (Wedges) on the

front and rear of tracks.

6.     Affix chains or ropes securely to the lower

framework of machine, in order to prevent the side slip of machine.

7.     Fix the bucket by chains or ropes.

Important: A wood board can be placed beneath  the bucket to prevent damaging the  ground by the bucket.

Safety Measures Taken during Transport

ΔWarning

● During the transport of machine,  understand   and abide by all applicable  safety rules, vehicle codes, and traffic  regulations.

● Take the length, width, height, and weight  of the transport truck with loaded machine

into consideration, in order to select the  best transport route.

● During the transport, do not start up or stop the transport truck suddenly or drive at high

speed. Otherwise, it will result in movement  or unbalance of loaded machine.

4-6

Maintenance

5-1

Maintenance Overview

Overview

Maintenance Overview

To maintain the good status and long-term

serviceability of the machine, please fulfill the  checking and maintenance correctly and safely abide by the procedures recommended by this  manual.

Based on the total operating time of the

machine, the checking and maintenance items can be divided into several groups: Every 10h (Walk-around checking and daily routine

checking), every 50h, and every 250h. Please   refer to the reading of hour meter to determine the checking and maintenance timing. The

items for which the checking and  maintenance intervals can’t be determined are listed in

column “As necessary” .

When the machine is operated in extremely

severe environment (Dusty or high temperature

environment), fulfill the checking and

maintenance ahead of the periods specified by maintenance schedule.

Maintenance Precautions

Do not fulfill any other checking or

maintenance item not listed in this manual.     For the items not listed in this manual, please ask your sales or service dealer for help.

Keeping Machine Clean

●      Clean the machine before checking and

maintenance. Keep the machine clean.

●      Stop the engine before cleaning the

machine. Cover the electric parts against     water ingress. The water ingress into the     electric parts will probably result in  short- circuit or malfunction. Do not clean  the

battery, electronic control units,  sensors,  connector, or operating room by  water or steam.

Fuel, Lubricating Oils, and Greases

●      Select fuel, lubricating oils, and greases as per the “Fuel and Lubricant Table” .

●      Use the water-free fuel, lubricating oil, and grease. Take cautions to eliminate the

ingress of dusts during replacement or

filling of fuel, lubricating oil, and grease.

●      Store the fuel, lubricating oil, and grease in designated location and guard against the    ingress of water and dusts.

Precautions for Refueling

●      If the fuel filler port is installed with a filter screen, do not remove the filter   screen during refueling.

●      Please ensure to tighten the fuel tank cap after refueling.

●      The refueled volume shall not exceed the designated fuel volume.

No Cleaning of Engine Parts by Fuel    It’s prohibited to clean engine parts by fuel. Use

5-2

Maintenance Overview

non-inflammable detergent.

Guard against Ingress of Dusts

The installation and disassembling of parts shall be operated in a dust-free place. Clean the

working area and clean the parts to guard against ingress of dusts.

Cleaning of Mounting Surfaces

Keep clean the contact surface of parts during     the installation and disassembling. If the sealing grooves of the contact surfaces are damaged,

please contact your sales or service dealer for repairs or breakup.

Seal Rings and Split Pins

●      Ensure to replace all disassembled seal rings and split pins with new ones.

●      Take cautions not to damage or twist any seal ring during installation.

Seal Rings

Leave 1~2 turns of threads not wrapped with seal ring.

Seal rings

●      While wrapping the screw plug with seal   ring tape, thoroughly remove the used seal ring tap from the threads and clean the

threads.

●     Wrap the threads with seal ring and notice  to leave 1~2 end threads of screw plug not wrapped with seal ring.

Waste Disposal

●      Ensure to collect the drained oil of

machine into a container. The improper treatment of waste oil will pollute the    environment.

●      While disposing harmful objects,

including lubricating oil, fuel, coolant, filter, and battery, please abide by the   applicable laws and regulations.

Checking after Maintenance

●      Accelerate the engine speed gradually from idling speed to fastest speed and check for leakage of oil or water from repaired parts.

●      Operate all joysticks and check machine for normal operations.

Precautions for Wire Connections of Battery

●      Before operating the electronic system or performing electric welding, disconnect

the wires from both electrodes (+ and -) of the battery.

●      Ensure to disconnect it from the grounding electrode (-). Finally connect the

grounding electrode during connection.

●      Do not disconnect the battery wires during the normal running of engine. Otherwise,   the rotary converter circuit or other parts    will probably be damaged.

5-3

Maintenance

Service Data

Service Data

Fuel and Lubricant Table

Please refer to following table to select appropriate fuel, lubricating oil, and grease based on the temperature.

●      Regardless of the specified periods, replace the oil when the oil is too dirty or already deteriorated.

●     Never mix the oils of different trademarks while adding fuel/oil. To replace with fuel/oil of other trademark, please replace completely.

Fuel

Specified fuel

The diesel shall meet the following specification. This table lists some diesel specifications  available

in the world.

| Diesel specification | Region | Diesel specifications | Region |
| --- | --- | --- | --- |
| GB252 | China | JIS K2204, grade 2 | Japan |
| ASTM D975 No. 1-D, S15 No. 2-D, S15 | USA Canada | ISO 8217DMX | Worldwide |
| Bio-diesel Bio-diesel mixture B5 ASTM D6751, D7467 |  |  |  |
| EN590 : 2009 | EU | BS2869-A1 or A2 | UK |
| Bio-diesel Bio-diesel mixture B5 EN14214, EN590 |  |  |  |

5-4

Maintenance

Service Data

| Fuel tank | Diesel | To maintain the performance and life of engine, please always use clean and high-quality fuel. ● To prevent freezing in cold weathers, please choose diesel still suitable when the actual temperature is less by at least 12℃ than the expected minimum outside temperature. ● Please use diesel with cetacean number at 45 or higher. During the operations in low temperature or high altitude regions, use fuel of higher cetacean number. ● Use fuel with sulfur content (volume ratio) at <0.5%. Use ultra-low sulfur fuel especially in U.S. and Canada. The use of fuel with high sulfur content can probably result in sulfur acid corrosion in engine cylinders. ● Do not mix the diesel with kerosene, used engine oil, or residual fuel. It’s prohibited to use kerosene. ● The low quality fuel will reduce the engine performance and/or damage the engine. ● It’s not recommended to use fuel additives. Some fuel additives will deteriorate the engine performances. |
| --- | --- | --- |
|  |  | Safety measure for use of bio-diesel The warranty regulation of the engine manufacturer is void for the engine using disqualified or deteriorated bio-diesel. |

5-5

Maintenance

Service Data

Lubricant

| Position | Type | Type depending on temperature -20 -10 0 10 20 30 40℃ |  |  |  |  |  |  |  |  |  |  |  | Replacement period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oil sump | Diesel engine fuel API: grade CD ACEA:E-3,E-4 or E-5 |  | SAE 10W-30 |  |  |  |  |  |  |  |  |  |  | First 50h * Afterwards every 250h |
|  |  |  |  | SAE 15W-40 |  |  |  |  |  |  |  |  |  |  |
| Hydraulic oil tank | Anti-wear hydraulic oil |  |  |  |  |  |  |  |  |  |  |  |  | Every 2000h *** |
|  |  |  | ISO VG32 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ISO VG46 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ISO VG68 |  |  |  |  |  |  |
| Cooling system | Coolant (Water + coolant) ** SAE: J814C or J1034 | 50% coolant mixture |  |  |  |  |  |  |  |  |  |  |  | Every 1000h |
|  |  |  |  |  | 30% coolant mixture |  |  |  |  |  |  |  |  |  |
| Traveling reducer gear | Gearbox API: GL-4 | SAE 90 |  |  |  |  |  |  |  |  |  |  |  | First 250h * Afterwards every 1000h |
| Slewing motor gear | Lithium-base grease EP-2 MLGI 2# | -- |  |  |  |  |  |  |  |  |  |  |  | Every 50h |
| Slewing bearing |  |  |  |  |  |  |  |  |  |  |  |  |  | Every 50h |
| Working device |  |  |  |  |  |  |  |  |  |  |  |  |  | Daily or every 10h |
| Arm |  |  |  |  |  |  |  |  |  |  |  |  |  | As necessary |

*:     If the traveling time accounts for a high ratio of the total operating time, replace the gear oil ahead of the specified periods.

**: Use running water (soft water). Do not use well water or river water. If the outside temperature  is <0℃, add coolant (antifreeze). Determine the mixture ratio as per the coolant manufacturer’s

instructions.

***: The replacement period of hydraulic oil depends on the type of hydraulic oil in use. The new

machine is filled with Ken stone genuine hydraulic oil 46 at the time of delivery and the

hydraulic oil replacement periods described in this manual are based on the use of Ken stone

genuine hydraulic oil 46. When common anti-wear hydraulic oil is used, replace the hydraulic oil once every 2,000h.

API: American Petroleum Institute

ACEA: Association eds Constructors Europeans SAE: Society of Automotive Engineers

5-6

Maintenance

Service Data

Periodical Replacement of Hydraulic Oil

With installed hydraulic breaking hammer, the hydraulic oil deteriorates faster than the

common digging operations. Ensure to replace

the hydraulic oil and oil return filter element.

●      The failure to replace timely will result in  damage of machine and breaking hammer hydraulic system. To prolong the lives of   hydraulic devices, please timely replace     the hydraulic oil and oil return filter

element as per the table below.

●      During the replacement of hydraulic oil, clean the oil suction filter screen.

Replacement period (Hour)

| Item | Hydraulic oil | Filter element |
| --- | --- | --- |
| 1st time | -- | 25 |
| 2nd time | -- | 100 |
| Periodic | 1200(600) | 200 |

By taking the breaking hammer operating ratio of 100% for instance. Refer to “Hydraulic

Breaking Hammer” in page 8-6 for details.

Replacement period (Hour)

Hydraulic device   replacement period

Filter replacement period

Breaking hammer operating ratio (%)

( ): For the use of common anti-wear hydraulic oil.

5-7

Maintenance

Service Data

List of Wearing Parts

Periodically replace the wearing parts, including filters and filter elements, as per the table  below.

| System | Item | Part name | Replacement period |
| --- | --- | --- | --- |
| Hydraulic system | Return oil filter |  | First 50 hours for new machine or every 500 hours |
| Hydraulic system | Ventilator |  | Every 1000 hours |
| Engine lubrication system | Engine oil filter | Filter cartridge | Replace after first 50 hours, then every 250 hours |
| Fuel system | Fuel filter | Filter cartridge | Every 250 hours |
|  | Water separator |  |  |
|  |  | filter |  |
|  |  |  | Every 500 hours |
| Air filter system | Air filter | Primary (Outer) filter element | Every 1,000h or 6 cleaning cycles (Whichever comes first) |
|  |  | Secondary (Inner) filter element | At replacement of primary filter element |

5-8

Maintenance

Service Data

List of Tools (If installed)

| List of tools |  |  |  |  |
| --- | --- | --- | --- | --- |
| S/N 1 | Name and specification Tool box | Unit PCs | Quantity 1 | Remark |
| 2 | Socket hex. wrench (4mm) | Piece | 1 |  |
| 3 | Socket hex. wrench (5mm) | Piece | 1 |  |
| 4 | Socket hex. wrench (6mm) | Piece | 1 |  |
| 5 | Socket hex. wrench (8mm) | Piece | 1 |  |
| 6 | Socket hex. wrench (10mm) | Piece | 1 |  |
| 7 | Socket hex. wrench (12mm) | Piece | 1 |  |
| 8 | Stud dead spanner22*24 | Piece | 1 |  |
| 9 | Stud dead spanner 27*30 | Piece | 1 |  |
| 10 | Stud dead spanner 30*32 | Piece | 1 |  |
| 11 | Stud dead spanner 36*41 | Piece | 1 |  |
| 12 | Combination wrench, 8mm | Piece | 1 |  |
| 13 | Combination wrench 10mm | Piece | 1 |  |
| 14 | Combination wrench 12mm | Piece | 1 |  |
| 15 | Combination wrench 13mm | Piece | 1 |  |
| 16 | Combination wrench 14mm | Piece | 1 |  |
| 17 | Combination wrench 15mm | Piece | 1 |  |
| 18 | Combination wrench 16mm | Piece | 1 |  |
| 19 | Combination wrench 17mm | Piece | 1 |  |
| 20 | Combination wrench 18mm | Piece | 1 |  |
| 21 | Combination wrench 19mm | Piece | 1 |  |
| 22 | Socket head, 10mm | Piece | 1 |  |
| 23 | Socket head 11mm | Piece | 1 |  |
| 24 | Socket head 12mm | Piece | 1 |  |
| 25 | Socket head 13mm | Piece | 1 |  |
| 26 | Socket head 14mm | Piece | 1 |  |
| 27 | Socket head 15mm | Piece | 1 |  |
| 28 | Socket head 16mm | Piece | 1 |  |
| 29 | Socket head 17mm | Piece | 1 |  |
| 30 | Socket head 18mm | Piece | 1 |  |
| 31 32 | Socket head 19mm Socket head 21mm | Piece Piece | 1 1 | Extended socket head |
| 33 | Socket head 22mm | Piece | 1 |  |
| 34 | Socket head 24mm | Piece | 1 |  |
| 35 | Socket head 27mm | Piece | 1 |  |
| 36 37 38 39 | Socket head 30mm Curved extension rod, 12.5mm Short extension rod, 12.5mm Wire cutter, 200mm | Piece Piece Piece Piece | 1 1 2 1 | Changed to 30mm One long and one short each |
| 40 | Nipper pliers, 200mm | Piece | 1 |  |
| 41 | Ratchet wheel handle, 12.5mm | Piece | 1 |  |
| 42 | Adjustable wrench, 300mm | Piece | 1 |  |
| 43 | Ball hammer, 1.5pon | Piece | 1 |  |
| 44 | Straight screwdriver, 200mm*8mm | Piece | 1 |  |
| 45 | Phillips screwdriver, 200mm*8mm | Piece | 1 |  |
| 46 | Grease gun, 400g | Piece | 1 |  |
| 47 | Belt wrench, 60-140mm | Set | 1 |  |

5-9

Maintenance

Service Data

Table of Tightening Torques Nuts and Bolts (Grade ISO10.9)

Unless otherwise specified, tighten the nuts and bolts to the torques listed in table below.

●      The tightening torques for installation of plastic caps are not listed in the table below. To understand more details, please consult with your sales or service dealer. The excessive   tightening torque will cause damages.

●      If the replacement is required, replace with nuts and bolts of same dimensions and  specifications.

| Category | Width across flats (b) | Dimension (a) X pitch | Tightening torque |  |
| --- | --- | --- | --- | --- |
|  |  |  | Common connection |  |
|  | mm | mm | N·m | Ft-lb |
| Coarse thread | 10 | M6 X 1.0 | 9,8 ± 0,5 | 7, 2±0,4 |
|  | 12, 13 | M8 X 1.25 | 22,6±1,1 | 16,6±0,8 |
|  | 14, 17 | M10 X 1.5 | 47,1±2,4 | 34, 7 ±1,7 |
|  | 17,19 | M12 X 1.75 | 83,4±4,1 | 61,5±3,0 |
|  | 19, 22 | M14 X 2,0 | 134,4±6,7 | 99,1±4,9 |
|  | 22, 24 | M16 X 2.0 | 207,9±10,4 | 153, 3 ± 7,7 |
|  | 27, 30 | M20 X 2, 5 | 410,9±20,5 | 303,1 ± 15,1 |
| Fine thread | 12, 13 | M8 X 1.0 | 24,5±1,2 | 18,1±0,9 |
|  | 14, 17 | M10 X 1.25 | 50±2, 5 | 36,9±1,8 |
|  | 17, 19 | M12 X 1.5 | 87,3±4,3 | 64,4 ± 3,2 |
|  | 19, 22 | M14 X 1.5 | 135, 3±6,8 | 99,8±5,0 |
|  | 22, 24 | M16 X 1.5 | 220,6±11 | 162,7 ± 8,1 |
|  | 27, 30 | M20 X 1.5 | 452, 1 ±22,6 | 333,4 ± 16,6 |

5-10

Maintenance

Critical Safety Parts

Critical Safety Parts

To operate the machine safely, please fulfill the periodical checking and maintenance. The following critical safety parts shall be replaced periodically to improve safety. These parts can cause serious

injuries or fire accident if they are damaged. List of Critical Safety Parts

| Vehicle body |  | Periodically replaced critical safety part | Replacement period |
| --- | --- | --- | --- |
| Fuel system |  | Fuel pipes | Every 2 years |
|  |  | Packing on fuel tank cover |  |
| Heater |  | Heater hoses |  |
| Cooling system |  | Rubber hoses |  |
| Hydraulic system | Main machine | Hydraulic pipe (Pump outlet) |  |
|  |  | Hydraulic pipe (Pump oil suction port) |  |
|  |  | Hydraulic pipe (Slewing motor) |  |
|  |  | Hydraulic pipe (Traveling motor) |  |
|  | Working device | Hydraulic pipe (Boom cylinder pipeline) |  |
|  |  | Hydraulic pipe (Bucket arm cylinder pipeline) |  |
|  |  | Hydraulic pipe (Bucket cylinder pipeline) |  |
|  |  | Hydraulic pipe (Swing cylinder) |  |
|  |  | Hydraulic pipe (Bulldozing blade cylinder) |  |
|  |  | Hydraulic pipe (Span cylinder) |  |
|  |  | Hydraulic pipe (Pilot valve) |  |
|  |  | Hydraulic pipe (Auxiliary pipeline) |  |
|  |  | Seat Belt | Every 3 years |
|  |  | Anti skid plate |  |

5-11

Maintenance

Critical Safety Parts

The materials of the above-mentioned critical safety parts will deteriorate along with time to cause

wear or deteriorated performance. It’s difficult to determine the deterioration extent during periodical    checking. Therefore, such parts shall be replaced with new ones to guarantee stable performance,  even if such parts are still working well. Please be noted that, regardless of the replacement schedule,   any     part with wear symptom must be replaced immediately. Upon detection of any deformed or   cracked

pipe clamp, immediately replace it together with hose. Please consult with your sales or   service dealer for the replacement of safety parts.

Except  the  critical  safety  parts,  check  and tighten the  hydraulic  pipes and  when necessary  replace. During the replacement of hydraulic pipes, replace the O-ring and seal ring as well.

Check the fuel and hydraulic pipes as per the schedule specified by the following table. Refer to “Maintenance” .

| Checking type | Checking item |
| --- | --- |
| Daily routine checking | Leakage of hydraulic or fuel pipe connector |
| Monthly checking | Leakage of hydraulic or fuel pipe connector Damage (Cracking, wear, and tear) of hydraulic or fuel pipe |
| Yearly checking | Leakage of hydraulic or fuel pipe connector Aging, distortion, and damage (Cracking, wear, and tear) of hydraulic or fuel pipe and/or status of hose in contact with other machine parts |

5-12

Maintenance

Maintenance List

Maintenance List

| Checking and maintenance item | Page |
| --- | --- |
| Walk-Around Checking |  |
| Opening of engine hood and machine hood for checking | 5-16 |
| Walk-around checking of machine | 5-16 |
| Checking in driver seat | 5-16 |
| Daily routine checking (Every 10h) |  |
| Checking and adding of coolant | 5-17 |
| Checking and adding of engine oil | 5-18 |
| Checking of fuel level | 5-18 |
| Checking of hydraulic oil tank oil level and adding of oil | 5-19 |
| Lubrication of Working Device | 5-21 |
| After first 50h (For new machine only) |  |
| Checking and adjustment of fan belt | 5-23 |
| Replacement of hydraulic oil return filter | 5-24 |
| Every 50h |  |
| Checking and adjustment of track tension | 5-25 |
| Lubricate slewing support and slewing motor gears | 5-26 |
| Water drainage from fuel tank | 5-27 |
| Checking of battery electrolyte level and adding of electrolyte | 5-28 |
| Every 250h |  |
| Checking and adjustment of fan belt | 5-30 |
| Replacement of engine oil and filter | 5-30 |
| Cleaning of air filter | 5-31 |
| Checking of accelerator control system | 5-32 |
| Cleaning of radiator fins and oil cooler fins | 5-32 |
| Replacement of fuel filter | 5-33 |
| Every 1000h |  |
| Replace the air filter | 5-35 |
| Check and adjust the engine valve clearance | 5-35 |
| Clean the engine cooling system | 5-36 |
| Every 1500h |  |
| Check and clean the engine fuel injector | 5-39 |
| Check the crankcase vent system | 5-39 |
| Every 2000h |  |
| Overlap engine valve seat | 5-40 |
| Replace hydraulic oil and clean suction filter | 5-40 |

5-13

Maintenance

Maintenance List

Maintenance List

| Checking and maintenance item | Page |
| --- | --- |
| when needed |  |
| Replace the teeth | 5-42 |
| Replace the bucket | 5-43 |
| Lubricate Joysticks | 5-44 |
| Check the rubber track | 5-45 |
| Replace the rubber track | 5-47 |

5-14

Maintenance

Walk-around Checking

Walk-Around Checking

Fulfill the following checking before the first start of engine each day.

|  | Warning |  |
| --- | --- | --- |

●     Before operations, fulfill the walk-around checking and when necessary  repair immediately.

●     Before operations within the machine, please securely fix the engine hood or machine hood.   Keep the engine hood and machine hood closed under windy weather or while parking  the machine on a slope.

Before starting the engine, patrol around the machine and remove any combustible materials

around the engine. At the same time, check the machine for oil or water leakage, as well as loose or damaged nuts, bolts or wires.

5-15

Maintenance

Walk-around Checking

Opening of engine hood and machine            7.      Check traveling motor for oil leakage.

hood for checking                                           8.      Check shelter and guard plates for damage

1.     Check for presence of branches, leaves, oil,                    and check for loose nuts and bolts.

and other inflammable around the engine               9.       Check exterior and interior rear view

and battery.                                                                            mirrors for contamination and damage and

2.      Check for presence of lubricating oil and                       when necessary adjust.

coolant leakages around the engine.                           10. Check labels for presence of dusts and

3.      Check hydraulic oil pipes, hydraulic                                damage.

devices, hoses, and connectors for oil                       Checking in Driver Seat

leakage.                                                                        1.      Check seat and seat belt for presence of

Walk-around checking of machine                        dust and damage.

1.      Check lamps for presence of dusts and                      2.       Check driver seat for presence of dust, oil

damages and check lamp bulbs for                                   dirt, and other inflammable.

burnout.                                                                        3.      Check monitor, electric devices, and

2.      Check hydraulic accessories and hoses for                     switches for oil dirt, dust, and damage.

damage.

3.      Check bucket, bucket teeth, and side teeth

for wear, damage, and looseness.

4.      Check hook, anti-slip stop block, and hook

seat on bucket for presence of damage. (Optional)

5.      Check handrails, footplates, and  anti-slip

surfaces for damage and check for any loose bolt.

6.      Check tracks and track shoes for presence

of slip, check track carrier rollers, idling device, and sprocket for presence of

damage and wear, and check for loose bolt.

5-16

Maintenance

Daily routine checking (Every 10h)

Daily routine checking (Every 10h) Fulfill the following checking before the first

start of engine each day.

|  | Warning |  |
| --- | --- | --- |

●      Before operations, fulfill the daily

routine checking and when necessary repair immediately.

●      Before operations within the machine,  please securely fix the engine hood or   machine hood. Keep the engine hood    and machine hood closed under windy weather or while parking the machine  on a slope.

Checking and adding of coolant

|  | ΔWarning |  |
| --- | --- | --- |

●     Do not disassemble the radiator cap or drainage plug when the coolant is hot.

Stop the engine, wait for the engine and   radiator to cool down, and then slowly     loosen and take out the radiator cap and drainage plug.

●     While handling the coolant  (antifreeze), ensure to wear goggles and gloves. In

event of splashing of coolant (antifreeze) into your eyes or onto your skin, clean     by clean water immediately. Otherwise

it will cause injuries.

1.     Open the engine hood.

2.     Check the coolant level in the water tank

(1). The coolant level shall be within upper limit and lower limit. If below the  lower     limit, add coolant.

Adding

1.     Take out the cap of water tank (1).

2.     Add coolant, till the coolant level reaches

upper limit of water tank (1).

Upon detection of empty water tank (1)

during checking, check for presence of

water leakage and then check water level   of radiator (2). When necessary, add water to the radiator (2) and then add water to

the water tank (1).

3.     Install the cap (1).

Note: Add clean water (soft water) only to

compensate the lost coolant due to evaporation. Add the coolant (antifreeze) and clean water

(soft water) of specified mixing ratio only to

5-17

Maintenance

Daily routine checking (Every 10h)

|  | ΔWarning |  |
| --- | --- | --- |

compensate the lost coolant due to leakage. Checking and adding of engine oil

Please stop the engine and wait for the

machine to cool down before maintenance. .

Checking

1.     Open the engine hood.

2.     Withdraw the engine oil dipstick (1) and

wipe it dry by a rag.

3.     Insert the oil dipstick (1) to the end and

then withdraw again.

4.     Check the oil on the oil dipstick (1). The

oil level shall be within upper limit and

lower limit. If below the lower  limit, add oil.

Adding

1.     Take out the oil filler cap (2).

2.     Add oil till the oil level is within upper

limit and lower limit of oil dipstick  (1).    Both excessive and insufficient oil  levels will cause malfunction.

3.     Tighten the oil filler cap (2).

4.     Start the engine, run at low idling speed

for 5min, and then stop the engine.

5.     Check the oil level after approximately

10min.

Checking of fuel level

|  | ΔWarning |  |
| --- | --- | --- |

●     Do not smoke or use open fire while handling fuel or performing the

operations of fuel system.

●     Do not disassemble the fuel tank cap or refuel while the engine is running or is   not cooled down. Do not splash fuel to    any high temperature surface of

machine.

●     Refuel the fuel tank in a well-ventilated place.

●     Any overflown fuel shall be wiped away immediately.

●     Do not top up the fuel tank. Leave an expansion space for the fuel.

●      Tighten the fuel tank cap securely.

●      Use correct fuel grade based on the

season.

5-18

Maintenance

Daily routine checking (Every 10h)

Add fuel

1.     Check the fuel level by fuel gauge .

2.     If the fuel level is low, open the fuel tank

cap and add fuel.

Checking of hydraulic oil tank oil level and adding of oil

|  | ΔWarning |  |
| --- | --- | --- |

Disassembling cap or filter or disconnecting pipeline before the pressure relief of

hydraulic system will probably result in  out- spray of hydraulic oil.

●      Slowly loosen the ventilation plug to relieve the internal pressure of tank.

Checking

The oil level varies along with the oil

temperature. Maintain the machine and check the fuel status by the posture shown in the

diagram below.

●     Posture of machine for checking of hydraulic oil level

1.     Start the engine and run at low speed.

2.     Fully extend the track width.

3.     Fully retract the cylinders (Bucket arm and

bucket cylinders) and lower the bucket onto the ground.

4.     Lower the bulldozing blade and then stop

the engine.

5.     Check the oil level by level gauge.

When the oil temperature is approximately

20℃: The oil level shall be within upper limit and lower limit. If below the lower limit, add  oil.

When the oil temperature is approximately

50~80℃: The oil level shall be slightly lower than  upper limit.

5-19

Maintenance

Daily routine checking (Every 10h)

Add hydraulic oil                                                Pressurization of Hydraulic Oil Tank

Important: While adding oil, the oil level  shall not be above the upper limit.   Otherwise it will   damage the hydraulic  circuit or cause oil spray.  If the oil level is too  high due to carelessness,     stop the engine,  wait for the hydraulic oil to

cool down, and  drain the excessive oil from oil drainage  port.

Important: Do not forget applying pressure  to the hydraulic oil tank, in order to prevent  the    damage of pump due to air inhalation. (If  no     ventilator is installed.)

1.     Open the engine hood.

2.      Slowly loosen the bleeding plug (2) to

relieve the internal pressure and then take out the plug.

3.      Add hydraulic oil, till the oil level is in the

middle of level gauge (1).

4.      Pressurize the hydraulic oil tank as shown

in diagram below.

1.     Start the engine and run at low speed.

2.      Disassemble the bleeding plug (2) and set

the safety lock handle to unlocking position.

3.      Fully extend the bucket, bucket arm,  boom,

swing, and span cylinders.

4.      Stop the engine and lift up the safety lock

handle to locking position.

5.      Tighten the bleeding plug and then

pressurize by retracting cylinders.

Note: For machines equipped with a

ventilator, there is no need to pressurize.

5-20

Maintenance

Daily routine checking (Every 10h)

Lubrication of Working Device

1.      Maintain the machine under the status shown in above diagram, lower the working device to the

ground, and then stop the engine.

2.      Lubricate the grease fitting by a grease gun.

3.      Wipe away excessive grease.

5-21

Records

5-22

Maintenance

After first 50h(for new machine only)

After first 50h (For new machine only) Checking and adjustment of fan belt

|  | Warning |  |
| --- | --- | --- |

Please stop the engine and wait for the

machine to cool down before maintenance. .

●      The engine, exhaust pipe, radiator,

hydraulic pipes, sliding parts, and many

other machine parts are really hot when the engine is just stopped. Touching

such parts can cause scalding.

Important: A loose belt can cause poor

charging of battery, overheating of engine, or earlier wear of belt. The over-tight belt will    damage the water pump or bearing and

alternator drive belt.

Important: Keep the belts away from all greases.

Checking

1.     Lift up and open the machine

cover.

2.   Check the tension (Approximately 98N)

based on the centers of fan pulley (1) and alternator pulley (2).

3.     Check the fan belt (3). Upon detection of

any of following conditions, replace.

●      Rupture or cracking.

●      The belt is worn to touch the bottom of pulley V-grooves.

●      The belt is stretched loosely and is  non- adjustable.

Adjustment

1.     Loosen bolt (5) and lock nut (4).

2.     Rotate the adjustment bolt (6) to move the

5-23

Maintenance

After first 50h(for new machine only)

alternator (7) and adjust the tension of fan belt (1).

Increase tension: Clockwise

Reduce tension: Counter-clockwise

3.      Tighten bolt (5) and lock nut (4).

Note: After the replacement with new belt, run the engine at low idling speed for

approximately 3~5min to run in new belt and then adjust the tension.

Replacement of engine oil and filter   Refer to page 5-30“Replacement of engine oil and filter ” .

5-24

Maintenance Every 50h

Maintenance Every 50h

Every 50h

Check and adjust track tension

ΔWarning

●       If it’s necessary to operate beneath the lifted machine or working device,

always use cushion woods, jack, or

other  firm and stable supports. Do not  access  the area beneath the machine or working device before it’s firmly

supported. This operation is especially important for the hydraulic cylinder    operations.

●       Guard against high pressure grease.

●       In the track pensioner, the grease is

injected under high pressure. If the

tension is not adjusted by the following designated procedure, the grease relief  valve will probably fly out to cause

injuries.

●       Never loosen the grease pipe fittings.

●       Slowly loosen the grease relief valve. Do not rotate it for >1 turn.

●       Do not place your face, arms, legs, or    body in front of the grease relief valve.

●       If no grease flows out when the grease relief valve is loosened, the valve is

malfunctioned. Please contact a  Ken    stone Heavy Machinery  service dealer for repairs.

Checking

1. For rubber tracks, adjust the marking “M”  at

the joints to the top center of track  frame.

2.     Lift the machine body by working device.

Slowly operate the joysticks.

5-25

Maintenance Every 50h

3.  Check  the  clearance  (A  or  B)  between  the

frame bottom face and track top face in the middle of track frame. The clearance (A  or

B) shall be within following range: (A)Rubber tracks

59~64mm

(B)Steel tracks 95 to 110mm

Adjustment

Increasing of Tension

1. Take out the cover plate (1).

2. Use a grease gun to inject lubricating grease

via the grease fitting (3) of  grease relief valve (2).

3. Check the track tension.

Reduce tension

1.      Remove the cover plate.

2.      Slowly loosen (by one turn) the grease

relief valve (2) by a wrench to drain the    grease. If it’s difficult to drain the grease, drive the machine back and forth.

3.      Tighten the grease relief valve (2).

•        Tightening torque: 177 N˙M

Lubricate slewing support and slewing motor gears

|  | Δ Warning |  |
| --- | --- | --- |

Do not slew during lubrication. Otherwise, you will probably be entangled into the

machine, which is really dangerous.

5-26

Maintenance Every 50h

|  | ΔWarning |  |
| --- | --- | --- |

1.     Stop the machine as per the posture shown

in above diagram and stop the engine.

2.     Lubricate the grease fitting by a grease

gun.

3.     Start the engine, lift the bucket, and rotate

clockwise for 90°.

4.     Lower the bucket onto the ground and then

stop the engine.

5.     Repeat above steps 2~4 for 3 times.

6.     Wipe away the grease drained from the

slewing bearing and grease pipe.

Water drainage from fuel tank

●     Do not smoke or use open fire while handling fuel or performing the

operations of fuel system.

●     Do not disassemble the fuel tank cap or refuel while the engine is running or is   not cooled down. Do not splash fuel to    any high temperature surface of

machine.

●     Refuel the fuel tank in a well-ventilated place.

●     Do not top up the fuel tank. Leave an expansion space for the fuel.

●     Any overflown fuel shall be wiped away immediately.

●      Tighten the fuel tank cap securely.

●      Use correct fuel grade based on the season.

5-27

Maintenance Every 50h

Start the machine only after draining.

1.     Slew the upper frame to a position easy  for

water drainage.

2.     Lift up and open the hood, Take out the fuel

tank cap (1).

3.     Place a box beneath the water drainage

plug (2).

4.     Loosen the water drainage plug (2) to fully

drain the water and the deposits from the tank.

5.     Tighten the water drainage plug (2).

6.     Tighten and lock the fuel tank cap (1).

7.     Exhaust air.

Air exhausting of Fuel System

Refer to page 6-8“Air exhausting from Fuel System ”.

Note: The air content in the fuel system will   cause start failure or start difficulty of engine. Bleed the air by same procedure described

above when the fuel tank is empty.

Checking of battery electrolyte level and adding of electrolyte

|  | Δ Danger |  |
| --- | --- | --- |

●     Do not use the battery when the

electrolyte level is below the lower limit.  Otherwise it will speed up the internal     aging and shorten the life of battery. In   addition, it can lead to burst (explosion).

●      The battery can generate inflammable  hydrogen to probably cause explosion.  Keep away from fire sources including open fire, spark, and ignited cigarette.

●      Clean the area above electrolyte level

marking by a wet cloth and check the

level. Do not clean by a dry  cloth,

otherwise    it   can cause electrostatic

accumulation      and     combustion      or explosion.

|  | ΔWarning |  |
| --- | --- | --- |

●     During the operations of battery, wear goggles and protective clothing.

●     Do not fill the distilled water till the

level is above the upper limit. Otherwise the electrolyte will leak out. The contact with such electrolyte can harm your

skin or corrode the machine parts.

●      The battery contains sulfur acid. The

accidental touch will harm your eyes  or

5-28

Maintenance Every 50h

skin.

•       In event of accidental contact with eyes, immediately  flush  by  clean  water  and quickly seek for medical treatment.

•       In event of accidental swallowing, drink a lot of water or milk and immediately seek for medical treatment.

•       If  the   sulfur  acid  comes  into  contact with your skin or clothing, immediately   flush by a lot of water.

Checking

1.     Open the maintenance cover.

2.      Check the fluid level. The fluid level must

be between upper limit (H) and lower limit (L). If not, add distilled water till the fluid  level reaches upper limit (H).

3.      Check terminals for looseness and

contamination.

Adding

If the adding of distilled water is required, add water before start of operations, in order to

prevent freezing.

1.      Take out the cap (1) and add distilled

water till the upper limit (H) is reached.

2.      Clean the ventilation port on the cap and

securely tighten the cap (1).

5-29

Maintenance   Every 250h

Every 250 hour

Checking and adjustment of fan belt Refer to page 5-23 “Checking and adjustment of fan belt ”.

Replacement of engine oil and filter

|  | Warning |  |
| --- | --- | --- |

Please stop the engine and wait for the

machine to cool down before maintenance. .

●      The engine, exhaust pipe, radiator,

hydraulic pipes, sliding parts, and many other machine parts are really hot when the engine is just stopped. Touching

such parts can cause scalding.

●      The engine oil is really hot. Take

cautions not to touch the hydraulic oil while loosening engine hood or

connector. Operating the machine in such case will cause scalding and

injuries.

1.     Open the engine hood and take out the oil

filler port (2).

3

1. Open the engine cover and remove the fuel filler opening (2).

2.Place a tray under the drain plug (3) for the waste oil.

3.Remove the oil drain plug (3) to drain the oil.                                Important: Check for metal powder in the waste oil. If it

contains a large amount of metal powder,  please consult your sales or service agent.

4.Press the oil drain plug (3) and tighten it with a wrench.

5.Use the oil filter wrench to turn the filter (4) counterclockwise to remove it.

6.Clean the installation surface of the filter housing.

7.Apply a small amount of oil to the gasket of the new filter.

8.Install the new filter manually.

9.After the filter gasket touches the mounting surface, tighten one more turn (use an oil

filter wrench). (Tightening torque with an

5-30

Maintenance   Every 250h

oil filter wrench: 19.6 to 23.5N·m).

10. Add lubricating oil between the upper

limit (H) and lower limit (L) of the oil    dipstick (1). If the oil level is too low or too high, it will cause problems.

11. Tighten the fuel filler cap.

12. Start the engine, run it at a low idle speed

for 5 minutes and turn off the engine.

|  | ΔWarning |  |
| --- | --- | --- |

13. After about 10 minutes, check the oil level. Cleaning of Air Filter

●      Please stop the engine and wait for  the machine to cool down before

maintenance.

•       The engine, exhaust pipe, radiator, and   many other machine parts are really hot when the engine is just stopped.

Touching them will cause scalding.

●     While using compressed air, please wear necessary protection appliances,

including goggles and filtration mask, as the metal fragments and other objects

will splash to cause accidents of serious harms.

Important: Take cautions not to scratch the  filter element. Do not use any damaged filter element.

Important: During the operations under  dusty environment, please daily fulfill the  checking   and maintenance for the machine.

Important: Ensure to steadily install the  filter element and dust cover. Otherwise, the  ingress of dusts into the cylinders will  damage the

engine.

1.     Open the engine hood.

2.     Loosen the clamp (1) and take out the dust

collector (2).

3.     Clean the inside of dust collector (2).

4.     Take out the primary filter element (3).

5.     Cover the inlet and outlet on the back of

5-31

Maintenance   Every 250h

body (4) by cloth or tape to guard against ingress of dusts.

<If the secondary filter element is installed>

To prevent the ingress of dusts into engine, do not take out the secondary                filter element (5).

6.     Clean the inside of housing (4).

7.     Clean primary filter element (3) by dry

compressed air (294~490kPa). Firstly,

blow along the wrinkles from the inside of filter element. Then blow from outside and finally blow from inside.

8.     Illuminate the inside of primary  filter

element (3) by a lamp to check. Upon     detection of any pinhole or dirt, replace.

9.     Remove the cloth or tape used in step 5.

10.   Install the primary filter element (3).

11.   Install the dust collector (2) by facing

upward the “↑TOP↑” and then secure it by clamp (1).

Checking of accelerator control system

●   The accelerator joystick and the speed

regulator  on the engine side are connected by wiring.  Check the wires at their fixing   positions for  presence of stretching and

looseness.

●   If the wires at their fixing positions are stretched or loose, there probably has a  malfunction.

●   The replacement and adjustment of wires

require experiences. Please ask your sales or service dealer for operations.

Cleaning of radiator fins and oil cooler

ΔWarning

While using compressed air, please wear    necessary protection appliances, including goggles and filtration mask, as the metal    fragments and other objects will splash to  cause accidents of serious harms.

Important: Take cautions not to damage the radiator fins during cleaning.

●     While using compressed air or high

pressure water, ensure that the pressure is no higher than 200kPa and keep the   air nozzle from the radiator for a

sufficient spacing.

Important: While cleaning by water, please cover the electric system against water

5-32

Maintenance   Every 250h

ingress.

Important: During the  operations under dusty environment, please  daily fulfill the checking   and maintenance  for the machine.

1.     Open the engine cover..

2.     Blow compressed air towards the radiator

|  | ΔWarning |  |
| --- | --- | --- |

fins to remove the adhered dusts and mud. Replacement of Fuel Filter

●     Do not smoke or use open fire while handling fuel or performing the

operations of fuel system.

●      Stop the engine in a  well-ventilated

place and wait for the machine to cool down before maintenance.

●     Any overflown fuel shall be wiped away immediately.

1

1.     Open the engine hood.

2.     Rotate the filter (1) counter-clockwise by a

filter wrench to take it out.

3.     Clean the mounting surfaces of filter

housing.

4.     Apply a thin film of fuel to the washer of

new filter housing.

5.     Manually install new filter.

6.     After  the  filter washer  comes  into contact

with   the   mounting   surface,   tighten   the filter further for one turn.

(Tightening    torque    by    filter    wrench:

20~24N.m).

7.     Exhaust the air.

Refer to page 6-8“Air Exhausting from Fuel System ”.

5-33

Records

5-34

Maintenance

Every 1000h

Every 1000h

Replace the air filter

|  | ΔWarning |  |
| --- | --- | --- |

Please stop the engine and wait for the

machine to cool down before maintenance. .

●      The engine, exhaust pipe, radiator, and   many other machine parts are really hot when the engine is just stopped.

Touching such parts can cause scalding.

Important: Do not use a wrinkled filter   element or a filter element with damaged washer or seal ring.

Important: Ensure to steadily install the  filter element and dust cover. Otherwise, the  ingress of dusts into the cylinders will  damage the

engine.

1. Open the engine hood.

2.     Loosen the clamp (1) and take out the dust

collector (2).

3.     Clean the inside of dust collector (2).

4.     Take out the primary filter element (3).

5.     Cover the inlet and outlet on the back of   body (4) by cloth or tape to guard against ingress of dusts.

6.     Clean the inside of housing (4).

7.     Remove the cloth or tape used in step 5.

8.     Install new filter element.

9.     Install the dust collector (2) by facing

upward the “↑TOP↑” and then secure it by clamp (1).

Checking and Adjustment of Engine Valve Clearance

These operations require experiences. Please    ask your sales or service dealer for operations.

5-35

Maintenance

Every 1000h

Cleaning of Engine Cooling System

|  | ΔWarning |  |
| --- | --- | --- |

●      Please stop the engine and wait for  the machine to cool down before

maintenance. .

•       The engine, exhaust pipe, radiator, and    many other machine parts are really  hot when the engine is just stopped.

Touching such parts can cause scalding.

•       The engine coolant is also under high

temperature and high pressure when

the engine is just stopped. Take special     cautions while loosening the cap or plug. Operating the machine in such case will  cause scalding and injuries by splashing  high temperature coolant.

●     If the maintenance requires the running of engine, assign two operators for

teamwork and keep contact with each other.

•       One operator must sit in the driver seat to get ready to stop the engine

immediately when necessary. This

operator must pay special attention not    to touch any joystick or pedal, unless it’s absolutely necessary.

•       The other operator for the maintenance must keep the body and clothing away   from motion parts of machine.

●     While the engine is running, it’s really     dangerous standing behind the machine, as the machine probably moves

suddenly. While the engine is running, never stand behind the machine.

●     Do not disassemble the radiator cap or drainage plug when the coolant is hot.

Stop the engine, wait for the engine and   radiator to cool down, and then slowly     loosen and take out the radiator cap and drainage plug.

During cleaning, if the coolant temperature is low, the thermostat will be closed and the

coolant will not flow within the radiator. Before cleaning, heat up the coolant to at least 90℃ .

1. Open the engine hood.

2. Slowly loosen the radiator cap (3) to  relieve

the internal pressure and then take  out the cap.

5-36

Maintenance

Every 1000h

3.     Place a tray beneath the water drainage

plug (4) to collect the used coolant and

then loosen the water drainage plug (4) to drain the coolant.

4.     Tighten the water drainage plug (4).

5.     Add running water through the water filler port into the radiator, till the water level

reaches the top of filler port. Add the water slowly and easily to prevent the ingress of   air into radiator.

6.     Install the radiator cap (3).

7.     Start the engine and run the engine at a

speed slightly higher than the low idling    speed. Heat up the water temperature to at least 90℃, open the thermostat, and then   run the engine for approximately 10min.

8.     Stop the engine, wait for the coolant to

cool down, and then take out the drainage plug (4) to drain the water.

9.     After water drainage, clean the cooling

system by detergent. Use the detergent as per the attached operation instructions of

the detergent.

10.   Repeat steps 4~8 to clean the cooling

system.

11.   Tighten the water drainage plug (4).

12.   Add new coolant (Mixture of antifreeze

and running water) through filler port to

the radiator, till the radiator is full. Add the coolant slowly and easily.

13.   Install the radiator cap (3).

14.   Warm up the engine. Use instruments to

check the cooling system for presence of abnormality.

15.   Increase the water temperature to at least

90℃ . Then, open the thermostat and run the engine for approximately 10min.

16.   Stop the engine, wait for the coolant to

cool down, and then check the coolant level in the radiator.

17.   Install the radiator cap (3).

18.   Clean the inside of water tank (1) and add

coolant, till the water level reaches upper limit (H).

19.   If the coolant is replaced, re-check the

coolant level after the operations of machine.

Once the machine starts operations, the     coolant is distributed to the whole system so that the coolant level drops.

Add the coolant to compensate the consumed coolant.

5-37

Records

5-38

Maintenance

Every 1500h

Every 1500-hour

Checking and Cleaning of Engine Fuel Injector Nozzles

These operations require experiences. Please    ask your sales or service dealer for operations.

Checking of Crankcase Ventilation Hole System

These operations require  experiences. Please   ask your sales or service dealer for operations.

5-39

Maintenance

Every 2000h

Every 2000h

Connection of Engine Valve Seats

These operations require experiences.Please ask your sales or service dealer for  operations.

Replacement of Hydraulic Oil and

Cleaning of Oil Suction Filter Screen

|  | ΔWarning |  |
| --- | --- | --- |

●      Please stop the engine and wait for  the machine to cool down before

maintenance. .

•       The engine, hydraulic system and most of parts are very hot when the engine    stalls. You may get scalded if you touch them.

•       The hydraulic oil is also under high

temperature and high pressure. Take

cautions not to touch the hydraulic oil

while loosening engine hood or

connector. Operating the machine under such condition will cause out-spray of

hot oil to result in scalding and

●     Disassembling cap or filter or

disconnecting pipeline before the

pressure relief of hydraulic system will probably result in out-spray of

hydraulic oil.

•       Slowly loosen the bleeding plug to relieve the pressure of oil tank.

•       While disassembling connector or plug    or disconnecting hose, stand on one side, loosen slowly to gradually relieve

internal pressure, and then take out.

1.     Adjust the machine to the hydraulic oil

level checking status.

Refer to page 5-19“Checking of hydraulic oil tank oil level and adding of oil ”.

2.     Lift up the safety lock handle to the

locking position.

3. Lift up and open the engine cover.

4. Slowly loosen the air plug (7) to reduce the internal pressure, and then lower the plug.

5. Place a tray under the drain plug (3) for waste oil.

6. Loosen the oil plug (3) and drain the hydraulic oil.

7. Loosen the hose strap and remove the hose from the flange (1).

8. Loosen the bolts and take out the flange (1).

5-40

Maintenance

Every 2000h

9. Loosen the bolt, remove the suction filter element (2), and clean it.

10. Clean the inside of the hydraulic oil tank.  11. Replace the bleed plug (7) (if equipped).    12. Install the suction filter element (2) on the flange (1).

13. Install the flange (1) back to its original position.

14. Tighten the oil drain plug (3).

15. Remove the rubber plug (6).

16. Fill the rubber tube (5) with hydraulic oil to the center of the round oil mark.

17. Install the hose back to the original position of the flange (1) and tighten it.

18. Pressurized hydraulic oil tank. (If not equipped with a breather, please refer to

"Pressurized Hydraulic Oil Tank" on page 5-20)

19. Install the rubber plug (6), and then install the bleed plug (7).

20. Exhaust the air from the hydraulic oil circuit as described in the "Exhaust" section below.

21. Adjust the machine to the hydraulic oil level check posture, and check the oil level after the    oil temperature drops.

(For details, please refer to "Checking of

hydraulic oil tank oil level   and adding of oil " on page 5-19)

Air Exhausting

Important: After the replacement of  hydraulic oil, bleed the air from hydraulic  pipelines and    hydraulic devices. Otherwise,  the hydraulic

devices will probably be damaged.

Cylinders

1.     Start the engine and run at low idling

speed for 10min.

2.      Keep the engine running at low idling

speed, extend and retract all cylinders for 4~5 times, but take caution not  to

telescope any cylinder to the end.

3.      Run the engine at high speed, extend and     retract all cylinders for 4~5 times, but take  caution not to telescope any cylinder to the end.

4.      Restore the engine to low idling speed,

extend and retract all cylinders for 4~5 times, and always telescope to the end.

5-41

Maintenance

As Necessary

As necessary

Replace bucket teeth

Replace the bucket teeth when the tooth sleeves are worn. Do not wait for the bucket to be

damaged before replacement.

|  | Warning |  |  |
| --- | --- | --- | --- |

●      Before the maintenance or repairs

beneath the machine, lower all movable working devices onto the ground or

lowest position.

●     During the repairs and replacements of  bucket teeth or side teeth, to prevent the accidental movement of machine,

securely fix the working device.

•       During the use of hammer, the pins and the metal fragments will probably fly out. It

will probably cause serious personal harms.

•       While hammering hard metal parts such as tooth pins, bucket teeth, side teeth, and

bearings, please wear protective articles including goggles and gloves.

•       While hammering tooth pins and bucket teeth, ensure there is no person in

surrounding area.

●      During operations, the non-authorized

personnel   are   prohibited   to   access   the

working zone.

If the teeth are worn down to 140 mm, replace them.

Disassemble

1. Clean the bucket and park the machine on a flat, firm and safe ground.

2. Lay the bottom of the bucket flat, and pad the bottom with spacers to take out the bolts (1).

3. Remove the starting key, and then check whether the bucket is stable.

4. Remove the bolt (1).

5. Replace the bucket teeth (2).

Installation

1.     Clean the area for installation of bucket

5-42

Maintenance

As Necessary

tooth. If there is any soil or mud in this area, the bucket tooth and tooth sleeve  probably can’t be inserted completely.

2.     Place the bucket tooth into the bucket.

3.     If the bucket toothis insecure, please insert a shim to reduce the gap.

4.     Insert the bolt from outer side of bucket

and tighten by nut.

•       Tightening torque: 102 N.m.

5.     Rivet the bolt by a press or perform  three-

point welding at the threaded portion  (Nut end).

Replacement of Bucket

|  | ΔWarning |  |
| --- | --- | --- |

●      Before the maintenance or repairs

beneath the machine, lower all working devices onto the ground or lowest

position.

●     If the maintenance requires the running of engine, assign two operators for

teamwork and keep contact with each

other.

•       One operator must sit in the driver seat to get ready to stop the engine

immediately when necessary. This

operator must pay special attention not    to touch any joystick or pedal, unless it’s absolutely necessary.

•       The other operator for the maintenance must keep the body and clothing away   from motion parts of machine.

●     During the use of hammer, the pins and the metal fragments will probably fly     out. It will probably cause serious

personal harms.

•       While hammering hard metal parts  such as tooth pins, bucket teeth, side teeth, and bearings, please wear

protective articles including goggles and gloves.

•       While hammering tooth pins and bucket teeth, ensure there is no person in

surrounding area.

●     While aligning pinholes, always check

by visual observation. Do not insert  your finger into pinholes,  otherwise

your finger will probably be lost.

5-43

Maintenance

As Necessary

Disassembling

1. Lower the bucket steadily onto the ground  as

shown in above diagram.

Note: While disassembling the tooth pin, adjust the bucket position in such manner that the

bucket barely comes into contact with the

ground. If the bucket is steadily on the ground, the resistance will be really high so that it’s

difficult to disassemble the tooth pin.

2. Place the safety lock handle to locking

position and stop the engine.

3. Unfasten the fastening nut (1) and remove it.

4. Use a hammer to knock the tooth pin (2) off

the bucket.

5. Remove the bucket.

Installation

②

Ⓑ

1.      Align the pin hole (A) on the bucket with

the pin hole on the stick ①, install the tooth pin and the fastening nut.

2.      2. Operate the cylinder, align the pin hole

(B) on the bucket with the pin hole on the   connecting rod ②, and install the tooth pin and the fastening nut.

Lubrication of Joysticks

|  | Warning |  |  |
| --- | --- | --- | --- |

Adjust the machine to stop status, stop the    engine, and withdraw and properly preserve the starter key. Otherwise, the machine will   probably move suddenly to cause serious

injuries or deaths.

In event of inflexible operation of any joystick, please add oil for lubrication.

5-44

Maintenance

As Necessary

Joysticks

1.Remove the lower base part of the sheath (1)

and turn it up.

2.Wipe off the old grease.

3.Spray grease to points (A) and (B). 4.Put the sheath (1) back in place.

5-44

Maintenance

As Necessary

Checking of Rubber Tracks                    ·  If the  status  of rubber tracks is  as  shown in the   diagram  below,  please  repair  or  replace.  For   detailed  repair  or  replacement,  please  consult   with your sales or service dealer.

Rubber Tracks

If  any  track   is   stretched   and   non-adjustable, please replace the track.

(1) Rubber bump

If the height (A) is ≤5mm, replace.

(2)    Wire rope

If two or more segments of the wire rope are exposed, please replace.

If more than half of the side face of wire rope is cut, please replace.

(3)    Metal core

Replace even if only one metal core falls off.

(4)    Rubber

If the  cracking  length  is  ≥60mm,  repair.  If the wire rope  is visible, repair  as  soon  as  possible, regardless of the cracking length.

≥60mm

5-45

Maintenance

As Necessary

Replacement of Rubber Tracks

Δ Warning

●   If it’s necessary to operate beneath the

lifted machine or working device,

always use cushion woods, jack, or other · firm and stable supports. Do not access     the area beneath the machine or

working device before it’s firmly supported.

●     If the maintenance requires the running of engine, assign two operators for

teamwork and keep contact with each other.

•       One operator must sit in the driver seat to get ready to stop the engine

immediately when necessary. This

operator must pay special attention not    to touch any joystick or pedal, unless it’s absolutely necessary.

•       The other operator for the maintenance must keep the body and clothing away   from motion parts of machine.

Disassembling

1.     Completely loosen the tension of rubber

tracks.

Refer to page 5-25 “Check and adjust track tension” .

2.     Lift the machine body by working device.

3.     Place a steel tube (1) in the rubber track

and rotate the sprocket slowly towards opposite direction.

4.     Rotate the sprocket till the steel tube (1)

closes to the pensioner and the rubber track is lifted off the pensioner and then stop the   rotation of sprocket.

5.     Laterally slide the rubber track to take out

it from track frame. Take out the other rubber track by same procedure.

5-46

Maintenance

As Necessary

Installation

1.      Lift the machine body by working device.

2.      Place rubber track on the sprocket.

3.      Place a steel tube (1) in the rubber track and rotate the sprocket slowly towards   opposite direction.

4.      Rotate the sprocket till the steel tube (1)

closes to the pensioner and the rubber track is lifted off the pensioner and then stop the  rotation of sprocket.

5.      Slide inward the rubber track to install it

onto the pensioner and then pull out the steel tube.

6.      Check rubber track for secure installation

on sprocket and pensioner.

7.      Tension the rubber track to standard

tension.

Refer to the “Checking and adjustment of track tension” in page 5-25 for details.

8.      Install the other rubber track by same

procedure.

5-47

Maintenance

Maintenance during Long-Time Parking

Maintenance during Long- Time  Parking

Parking Procedure

If it’s necessary to park the machine for ≥30  days, please park the machine indoors. If it’s necessary to park outdoors, please park the

machine on a wood-paved level ground and   cover the machine by water-proof cap to keep

dry.

1.     Clean the machine.

2.     Check for oil/water leakage and cracking

and check for any loose nut or bolt.

3.     Refuel and replace hydraulic oil and

lubricating oil.

4.     To prevent rusting and freezing, replace

the engine coolant with long-life coolant (LLC).

Refer to page 5-35 “Cleaning of radiator fins and oil cooler fins” .

5.     Lubricate the grease fitting by a grease

gun.

6.     Fully retract the bucket and bucket arm

cylinders and lower the bucket and bulldozing blade onto the ground.

7.     Apply anti-rusting oil to the hydraulic

cylinders.

8.     Disconnect the negative cable of battery

and cover the battery to guard against freezing.

During Parking

|  | ΔWarning |  |
| --- | --- | --- |

●     Do not operate the machine in an

enclosed place with poor ventilation.

●     If the natural ventilation is not possible, install ventilation fan, fan, extended

exhaust pipe, or other ventilation device.

1.     To prevent rusting, operate the machine

once a month to circulate the oil throughout whole system.

2.     Check the battery and when necessary

charge the battery. Please ask your sales or service dealer for charging.

Start of Machine after Parking

Important: If the above-mentioned

“Parking procedure” is not followed during  the long-time parking of machine, please  consult

with your sales or service dealer  before restarting the machine.

1.     Wipe away the lubricating oil from the

piston rods of the hydraulic cylinders.

2.     When necessary, add lubricating oil or

grease.

Reuse of Engine

1.     Fulfill the daily checking.

2.     Pte-lubricate the engine before start.

a. Crank the engine for 15s and notice  to

keep the fuel system closed to prevent start of engine.

5-48

Maintenance

Maintenance during Long-Time Parking

b.     Stop for 30s.

c.      Repeat this procedure, till the engine is already cranked for 1min in total.   This can enable the sufficient oil

circulation throughout the lubrication system of engine.

2.      Get ready the fuel system.

3.     Start the engine. Idle the engine for

approximately 15min and then:

•        Check for normal oil pressure.

•        Check for leakage of fuel, engine oil, and coolant and check indicator lamps and/or level gauge for normal functioning.

5. Avoid operating for a long time at  minimum   or maximum engine speed or  operating the loaded residue within the   first loading

hour.

5-49

Troubleshooting

6-1

Troubleshooting

Symptoms that are not malfunctions

Symptoms that are not malfunctions

The following symptoms are not malfunctions.   ·

●      With  engine  running  at  low  speed,  when the arm reaches a nearly vertical  position, its retraction speed will drop  shortly.

●      With engine running at low speed, when the bucket teeth reach a nearly horizontal

position, its movement speed will drop shortly.

●      A noise of slewing motor can be heard at the start and end of slewing.

●      A noise of traveling motor can be heard  when the machine stops suddenly during high speed traveling.

●     A noise of control valve can be heard     when the working device is subject to    excessive external force or moves to the end of travel.

●      The operations of the machine are more   labor some when an attachment heavier    than the standard bucket arm or bucket is installed.

6-2

Troubleshooting

Engine Overheating

Engine Overheating

|  | ΔWarning |  |
| --- | --- | --- |

●     If the steam overflows from the engine,  do not open the engine hood or machine hood. The steam or hot water will

probably spray out to cause scalding.

●     Do not disassemble the radiator cap or  drainage plug when the coolant is hot.    Stop the engine, wait for the engine and radiator to cool down, and then slowly   loosen the radiator cap to relieve the

internal pressure.

●      Please stop the engine and wait for the machine to cool down before

maintenance.

The following symptoms indicate overheating.

●      Engine water temperature should be <105℃

●      Steam came out of the engine room.

Remedy procedure

1.Park the machine on a safe place.

2.Check whether the steam overflows from the closed engine hood or machine hood.    3.If the steam overflows from engine

compartment, immediately stop the engine and contact your sales or service dealer for repair. If no steam overflows, run the

engine at low idling speed to cool down the coolant.

4.When the water temperature is less than 105°C, turn off the engine.

5.After the engine cools down,

immediately  take the following checking and remedy  measures.

Loose fan belt … Refer to page 5-23 for adjustment.

Low coolant level … Refer to page 5-17 for adding of coolant.

Water leakage … Repair.

Dirty radiator fins … Refer to page 5-32 for cleaning.

Deposits in cooling system … Refer to page 5-36 for cleaning.

If the malfunction still exists after  the above-     mentioned remedy procedures are taken,  please contact your sales or service dealers for  repairs.

6-3

Troubleshooting

No Battery Power

No Battery Power

The following symptoms indicate no battery

power.

●      No rotation of starter motor or start failure of engine.

●     Low volume of horn.

Remedy procedure

Start the engine by the booster battery on other machine (booster machine) and jumper cable.

|  | ΔWarning |  |
| --- | --- | --- |

●      To start the engine by jumper cable,

ensure to connect the cable by correct     procedure. The improper use of jumper cable will result in battery explosion or  unexpected machine motions.

•       The booster machine and the machine    with depleted battery are prohibited for mutual contact.

•       The positive (+) and negative (-) clamps of the jumper cable are prohibited for    mutual contact.

•       During connection, firstly connect  the jumper cable to positive (+) terminal.

During disconnection, firstly disconnect the cable from negative (-) terminal.

•       Connect the last clamp of jumper cable to a point as far as possible from the

battery.

cable to start engine.

Important: The jumper cable and clamps in use shall be suitable to the battery capacity. Do not    use any damaged or corroded jumper cable or

clamp.

Important: Ensure the same capacity for  the  battery on the rescue machine and the  battery on malfunctioned machine.

Important: Ensure to connect the cable clamps securely.

Connection of Jumper Cable

Important: Place the starter keys of both

rescue machine and malfunctioned machine  to

OFF.

To engine block

1.     Connect a clamp of jumper cable (R) to

the positive (+) pole of malfunctioned machine.

2.     Connect the other jumper cable clamp (R)

to the booster (+).

3.     Connect the jumper cable clamp (B) to the

charger (-).

●     Always wear goggles while using jumper           4.     Connect the other clamp of jumper cable

6-4

Troubleshooting

No Battery Power

(B) to the engine support of the

malfunctioned machine. Connect the

clamp to a point as far as possible from the battery.

After Start of Engine

1.      Check and ensure that the wire clamps are

already connected securely to various terminals.

2.      Start the engine of rescue machine and run

it at high speed.

3.      Start the engine of the malfunctioned

machine.

Disconnection of Jumper Cable

After the successful start of the engine of

malfunctioned machine, disconnect the jumper cable in reverse sequence of the connection

procedure.

Battery of rescue Battery of malfunctioned machine machine To engine block

from the charger (-).

3.      Disconnect the jumper cable (R) clamp

from the charger (+).

4.      Disconnect the jumper cable clamp (R)

from the uncharged machine (+).

Recharging

Please ask your sales or service dealer to recharge the depleted battery.

1.      Disconnect  the  clamp  of jumper  cable  (B)

from  the  engine  support of malfunctioned machine.

2.      Disconnect the other jumper cable (B)

6-5

Troubleshooting

Burnout of Fuse

Burnout of Fuse

If a lamp fails to turn on or the electronic

system fails to work, the fuse is probably burnt. Check the fuse.

|  | ΔWarning |  |
| --- | --- | --- |

Checking and Replacement of Fuses

If a fuse is burnt again shortly after

replacement, the electronic system is

probably malfunctioned. The improper

repair will probably cause fire danger. If the consultation is required, please contact your sales or service dealer.

1.      Rotate the starter key to OFF to stop the

engine.

2.     Open the engine hood.

1.      Open the fuse case cover (1).

2.      Check fuses for burnout.

Normal

Burnout

3.      If a fuse is burnt, please replace with spare

fuse of same specification.

Layout and protected circuits of fuses

| Current | Circuit |
| --- | --- |
| 20A | Rectifier |
| 30A | Start signal |
| 30A | Electric lock/preheat relay |
| 10A | Working lamp |
| 30A | Spare |
| 10A | Spare |

| Current | name |
| --- | --- |
| 12V60A | NO backup relay |
| 12V60A | NO start relay |
| 12V40A | NO charge relay |
| 12V40A | NO lamp relay |

6-6

Troubleshooting

Burnout of Fuse

Checking of Fuses

If the  machine  does  not   start  after  turning  the start  switch  to  the ON  position, it may  be  that the  starting  power  fuse  has  blown.  Open  the service  cover  and  check.  If the  fuse has blown, please contact your sales or service agent.

Precautions for external

maintenance power supply

Note: The external power supply is

limited to 12V15A. Can not be charged.

Note: The fuse is a large fuse used in high

current applications. Like ordinary fuses, they function as fuses, protecting electrical

components and wiring from damage caused by excessive current.

6-7

Troubleshooting

Restart after Refueling

Restart after Refueling

Air exhausting from Fuel System

Important: Do not rotate the engine by  starter motor to get ready the fuel system.  This will     probably cause overheating of  starter motor to  damage coil, pinion, and/or  ring gear.

Important: If the engine stops due to   shortage   of fuel, please add fuel, rotate the   key to ON for 60s, and then rotate it to   START position.

Running the starter motor  for a long time before the supply of sufficient  fuel will cause start

failure of starter

2.      Turn the starter key to the ON position and hold it for about 60 seconds..

Note: The air content in the fuel system will

cause start failure or other problems of engine. Ensure to exhaust when the fuel tank is  empty or the fuel system contains air.

1.     Add fuel.

6-8

Troubleshooting

Flashing of Warning Lamps

Flashing of Warning Lamps

If a warning tone is heard or a warning lamp starts to flash during operations, park the machine in a safe place and take the following remedy measures.

| Warning lamp | Lamp name | Cause and solution |
| --- | --- | --- |
|  | Charging warning lamp | There is a problem with charging. If the light continues to flash after maintenance, there may be a problem with the charger. If you need help, please consult your sales or service agent. |
|  | Engine oil pressure indicator lamp | The engine lubrication system is internally malfunctioned. Check the engine oil level. If this lamp flashes when the oil level is normal or still flashes after adding of oil, please consult with your sales or service dealer. Refer to page 5-18 “Checking and adding of engine oil ” . |

6-9

Troubleshooting

Other Symptoms

Other Symptoms

For the symptoms not listed in table below or the further existed problems after  appropriate solutions, please consult with your sales or service dealer.

| Problem | Cause | Solutions |
| --- | --- | --- |
| The left and right joysticks cannot move flexibly | ● Lack of grease on left and right joysticks | ● Lubricate the joystick |
| Throttle lever cannot move flexibly | ● The throttle line is loose | ● Adjust or replace (consult your sales or service agent). |
| Bucket, slewing or walking operations cannot be performed | ● The safety lock lever has been raised (locked) ● The travel lock adjustment is faulty | ● Lower (release) the safety lock lever. Please refer to page 2-11. ● Adjust the walking lock (consult your sales or service agent). |
| Not enough digging power | ● Hydraulic oil level is too low ● The hydraulic oil is not hot enough ● Air filter is clogged ● Incorrect hydraulic oil type | ● Add to the specified liquid level. Please refer to page 5- 17. ● Clean the air filter. Please refer to page 5- 27. ● Replace hydraulic oil. Please refer to page 5-34. |
| Unable to walk or inflexible | ● Stone or foreign object stuck | ● Remove foreign objects |
| The machine turns right/left | ● A stone or foreign object is stuck. ● The track tension adjustment is faulty | ● Remove foreign objects ● Adjust the track to the specified tension. Please refer to page 5-21. |

6-10

Troubleshooting

Other Symptoms

| Problem | Cause | Solutions |
| --- | --- | --- |
| Can't turn or turn inflexibly | ● Insufficient grease for slewing support | ● Lubricate the bearings Please refer to page 5-23 |
| The hydraulic oil temperature is too high | ● Hydraulic oil level is too low | ● Add to the specified liquid level. Please refer to page 5-17 |
| The starter motor rotates but the engine does not start | ● Insufficient fuel ● There is air in the fuel system ● Water in the fuel system | ● Add fuel. Please refer to page 5-16. ● Exclude air. Please refer to pages 6-8. ● Drain the water. Please refer to pages 5-24. |
| Tracks fall off | ● Tracks are too loose | ● Increase the tension. Please refer to page 5-21。 |
| The engine emits black smoke from time to time | ● Air filter is clogged | ● Clean the air filter. Please refer to pages 5-27。 |
| Engine exhausts white or blue smoke | ● Too much oil ● Inferior fuel | ● Adjust to the specified liquid level. Please refer to page 5-16. ● Change the fuel. |
| The engine produces irregular noise (combustion or mechanical noise) | ● Inferior fuel is being used | ● Replace the fuel |
|  | ● overheat of engine | For details, please refer to "If the Engine Overheats" on page 6-3. |
|  | ● Internal damage of exhaust pipe | ● Replace the exhaust pipe. For replacement matters, please consult your sales or service agent。 |

6-11

Troubleshooting Towing

Towing

|  | ΔWarning |  |
| --- | --- | --- |

During towing, the improper operations, the incorrect use of rope, or the improper

checking will cause serious injuries or deaths.

●      The breakage or disengagement of rope can cause danger. Use ropes compatible with the towing force.

●     Do not use any kinked, twisted, or even damaged rope.

●     Do not suddenly apply heavy load onto the rope.

●      Please wear safety gloves during the handling of rope.

●     Ensure that an operator is assigned each on the towing machine and the towed

machine.

●     Do not operate the towing operation on a slope.

●      Keep all personnel away from the  rope during towing.

Important: Do not tow the machine when the engine is not started or the machine is not

running. Otherwise, the towed machine will probably be damaged.

Important: Please ensure to abide by the

following procedure when the machine is  towed by traction hole. The negligence of any

of following steps will probably damage the traction hole or other framework part.

Towing of Machine

Use the following procedure to drag a heavy object or a machine entrapped in mud.

20° or less

●      Permissible force: 28.2KN

1.     Affix the rope to the hook ring (1).

2.     Hook the hook ring (1) to the traction hole

(2).

3.     Ensure that the rope maintains a 20° or

smaller angle with the traveling frame.

4.     Move the machine to tension the rope.

5.     Drive the machine at 2km/h or lower speed to a position (Easy for repairs)

within a short range from the site.

6-12

Parameter

7-1

Parameter

Basic Parameter

Applicable models: HFT11

| HFT11 | Technical parameters | Unit |  |  |
| --- | --- | --- | --- | --- |
| Overall weight |  | kg | 1123 | 1130 |
| Bucket capacity |  | m3 | 0.02 | 0.02 |
| Type of working device |  |  | Backhoe | Backhoe |
| Engine | Model |  | Kubota D902 | Yanmar3TNV74 |
|  | Displacement | L | 0.898 | 0.993 |
|  | Rated output power/speed | kw/r/min | 11.8/2300 | 11.2/2400 |
|  | Maximum torque/rotating speed | N.m/r/min | 51.3/1800 | 51.8/100 |
| Speed | Maximum traveling speed (High/low) | km/h | 1.5 | 1.3 |
|  | Slewing speed | rpm | 11 | 11 |
|  | Maximum grade ability |  | 30% | 30% |
| Track sub assembly | Ground pressure | k Pa | 34.64 | 29.9 |
|  | Material |  | Rubber | Rubber |
|  | Tension control type |  | Tightening cylinder | Tightening cylinder |
| Type of pump |  |  | gear pump | gear pump |
| Fuel/oil/fluid pressure and capacity | Working pressure | Mpa | 16 | 16 |
|  | Flow | (L/min) | 18.4 | 19 |
|  | Hydraulic oil tank capacity | (L) | 16 | 15 |
|  | Fuel tank capacity | (L) | 11 | 11 |

All ratings of the machine are obtained when the machine is operating on  a  solid horizontal  support surface.  If  the  working  environment  conditions  of  the  machine  differ  from  the  above-mentioned reference conditions (Such as the operations on uneven ground and slope), the operator shall take  these conditions into consideration.

7-2

Parameter

Overall Dimensions of Machine

Applicable models: HFT11

|  | Overall dimensions | Unit: mm |
| --- | --- | --- |
| a | Wheel track | 910 |
| b | Overall length of crawler | 1235 |
| c | Ground clearance of platform | 460 |
| d | Tail slewing radius of platform | 660 |
| e | Chassis width | 800-950 |
| f | Track width | 180 |
| g | Track height | 320 |
| h | Transport length | 2170/2850 |
| i | Ground clearance of seat | 1520 |
| j | Overall height | 2270 |
| m | Boom slewing - right | 50。 |
| n | Boom slewing – left | 55。 |

7-3

Parameter

Operating Range

Applicable models: HFT11

Working range                                         Unit: mm

| A | Ground maximum digging radius | 3040 |
| --- | --- | --- |
| B | Maximum digging radius | 3135 |
| C | Maximum digging depth | 1580 |
| D | Maximum digging height | 2620 |
| E | Maximum Dump height | 2000 |
| F | Maximum vertical digging depth | 1580 |
| G | Minimum slewing radius | 1510 |
| H | Maximum lifting height of bulldozing blade | 140 |
| I | Maximum digging depth of bulldozing blade | 200 |

7-4

Accessories

8-1

Accessories

General Safety Measures

|  | ΔWarning |  |
| --- | --- | --- |

General Safety Measures Safety Measures

Please take the following safety measures

while disassembling or installing attachments.

●      Please consult with Ken stone Heavy Machinery before installing optional products.

●     Do not use any attachment  not

approved by Ken stone Heavy

Machinery. Otherwise it will probably cause safety issues or cause adverse

influence on operations or life of machine.

●      Ken stone Heavy Machinery will not be liable for any injury, accident, or

product damage arising from the use of non-approved attachments.

●      Select a stable and level working zone. Ensure to park the machine in a  well- ventilated place.

●     Remove obstacles and dangerous   objects and immediately clear any splashed fuel.

●     During lifting, ensure to assign one

person as signaler. For detailed

procedure and measures, please  follow the instructions of the signaler.

●     If it’s necessary to place temporarily

heavy weights or accessories onto the

ground during disassembling or

installation, please ensure to place them in a steady place.

●     During lifting, keep all personnel away  from working zone. There is probably a danger of object falling or contact with  personnel in the working zone.

●      Use a crane to move heavy weights (≥25kg).

●     Ensure to properly support the heavy    weights before movement. While lifting a load by a crane, notice the gravity

center of the load and keep balance the machine.

●     Do not operate the machine when a load is lifted by a crane.

●      Follow the correct procedures to install   boom and bucket arm, otherwise it will   probably result in serious injuries. If the help is needed, please consult with your  sales or service dealer.

Precautions for Installation of Working Device

Ensure to fulfill commissioning after the

replacement with optional product or other special attachment. Check the hydraulic oil level and if necessary add oil. For detailed

8-2

Accessories

General Safety Measures

installation/disassembling procedures of

attachments, please consult with your sales or service dealer.

Precautions for Operations of Attachment

ΔWarning

The extended attachment will deteriorate the machine stability. While the machine is

traveling or slewing on a steep slope, the

imbalance will probably result in rollover. Do not take the following operations, otherwise it will be extremely dangerous.

Driving down a steep slope with attachment lifted

Driving laterally on a steep slope

Slewing on a steep slope

If a heavy attachment is installed, while

stopping the machine, the impulsion force of the machine will be larger than usual value   before the complete stop. Please carefully

take the distance into consideration to

prevent impact against surrounding objects. Keep a safe distance from surrounding

obstacles.

If a heavy attachment is installed, the gravity lowering (Slow lowering of attachment under the action of dead weight when the

8-3

Accessories

General Safety Measures

attachment is stopped in the air) will speed up.

The machine is more vulnerable to rollover in lateral direction than longitudinal

direction.

(1)   Do not slew laterally when the bucket working device is heavily loaded.

Especially, do not slow laterally on a slope.

(2)   For a machine with installed breaking hammer or grinder, the attachment is  heavier than the installed standard

bucket. Do not operate this machine   laterally, especially during the  down slope digging.

Please  carefully take the distance into

consideration  to prevent impact against

surrounding  objects of machine and keep a safe spacing  from surrounding obstacles.

8-4

Accessories

Precautions

Precautions for Operations of Attachment

|  | ΔWarning |  |
| --- | --- | --- |

●     Please consult with Ken stone Heavy Machinery before installing optional products.

●     Do not use any attachment not approved by Ken stone Heavy Machinery. Otherwise it will probably cause safety issues or cause adverse influence on operations or life of machine.

●     Ken stone Heavy Machinery will not be liable for any injury, accident, or product damage arising from the use of non-approved attachments.

●     If a bucket in width of >450M installed, the swing stop block must be installed. If no swing stop block is installed, it will probably result in impact of bucket with machine  body.

8-5

Accessories

Hydraulic Hammer

Hydraulic Breaking Hammer

For the operations of breaking hammer, please refer to the separately provided manual of

hydraulic breaking hammer.

ΔWarning

Do not use breaking hammer when the track width becomes narrower.

●     Always operate the machine with

extended track width of 950mm, in

order to improve the machine stability to the maximum extent. Operating the machine with narrow track width

(800mm) will probably result in rollover of machine due to poor stability.

●     If it’s necessary to operate the machine with narrow track width (800mm),

retract the bucket working device and lower the boom to reduce the gravity   center and face the  machine towards   front before traveling.

While operating the optional products such as breaking hammer, tipping bucket, and grab

bucket, ensure that the track width is extended to 950mm.

Important: While installing an attachment,

ensure that it’s suitable for the machine  model in use.

If the recommendations are required for the

choice of attachment, please contact your  sales or service dealer.

8-6

Accessories

Hydraulic Hammer

Precautions for Operations

●      Start the engine and run at 75% of maximum speed.

●      Perform the crushing operation by placing the hammer head perpendicular to the

working face.

●       During the crushing operations,

appropriately aim the hammer head at the object to be crushed and avoid the air

impact.

●   Do not pry by hammer head or pry during breaking.

●   Do not move the hammer head during crushing.

●   Do not impact one same surface continuously for >30s.

●

Do not crush when the cylinder is fully extended or retracted (at travel end).

Reserve at least 50mm travel.

8-7

Accessories

Hydraulic Hammer

●     Do not crush when the bucket arm is perpendicular to the ground.

●   Do not crush any object to be impacted by the gravity drop of breaking hammer.

●   Do not use the breaking hammer to move any object or rock to be crushed.

●   Slew the machine occasionally to cool down the engine.

●   In event of abnormal vibration of  hydraulic pipes, the nitrogen in the  accumulator is

probably leaking. Please  check as soon as possible.

Periodical Replacement of Hydraulic Oil

With installed hydraulic breaking hammer,  the   hydraulic oil deteriorates faster than  the normal operations. Ensure to replace  the hydraulic oil   and oil return filter  element.

●      The failure to replace timely will result in  damage of machine and breaking hammer hydraulic system. To prolong the life of     hydraulic system, ensure to replace the

hydraulic oil and oil return filter element after the hours listed in table below.

●      During the replacement of hydraulic oil, clean the oil suction filter screen.

Replacement period (Hour)

| Item | Hydraulic oil | Filter element |
| --- | --- | --- |
| 1st time | – | 25 |
| 2nd time | ― | 100 |
| Periodic | 1200(600) | 200 |

By taking the breaking hammer operating ratio of 100% for instance.

8-8

Accessories

Hydraulic Hammer

Replacement period (Hour)

Hydraulic oil

replacement period

Filter element

replacement period

hammer operating ratio (%)

Breaking

( ): For the use of common anti-wear hydraulic oil.

8-9

Records

8-10

Optional accessories

Auger                                                                               Ripper

Thumb part

Grab

Quick hitch

Rake

Digg ing bucket

200mm 300mm 500mm 600mm 700mm 800mm

900mm

600mm

Tilt bucket